Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2029 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF MACHINE ELEMENTS | # Max. marks : | # 100 |

(Use of DESIGN DATA book & Approved data sheets are permitted. Necessary data may be suitably assumed.)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | What are the different phases of the design process? | CO1 | 2 |
|  | b. | A mild steel bracket as shown in Fig.1 is subjected to a pull of 6000N acting at 45° to its horizontal axis. The bracket has a rectangular cross section whose depth is twice the thickness. Find the cross sectional dimensions of the bracket, if the permissible stress in the material of the bracket is limited to 60MPa. Fig. 1 | CO2 | 18 |
| (OR) |  |  |  |  |
| 2. | a. | Mention the various types of stresses induced in machine components. | CO1 | 2 |
|  | b. | A bolt is subjected to a shear load of 10 kN and tensile load of 25 kN. Determine the diameter of the bolt according to, i. Maximum principal stress theory, ii. Maximum shear stress theory, iii. Maximum principal strain theory. Assume factor of safety as 2.5, Yield point stress in simple tension as 300 N/mm2, poisson ratio: 0.25. | CO3 | 18 |
| 3. | a. | Define stress concentration and stress concentration factor. | CO1 | 2 |
|  | b. | What is a nip? Express its importance in the leaf springs. What is the other method which can be used instead of nipping? | CO1 | 4 |
|  | c. | A circular bar of 500 mm length is freely supported at its two ends. It is acted upon by a central concentrated cyclic loads having a minimum value of 20 kN and maximum value of 50 kN. Determine the diameter of the bar by factor of safety 1.5, size factor 1.85, surface finish factor 0.9. The material properties of bar are given as ultimate strength: 650 MPa, yield strength: 500 MPa, Endurance strength: 350 MPa. | CO3 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | A shaft running at 500 rpm transmit a power 10kW. Assume allowable shear stress as 640 N / mm2. Find the diameter of the shaft. | CO2 | 4 |
|  | b. | A truck has leaf spring of 10 leaves and is supported at a span length of 1m, with central band of 80 mm wide. A load of 6 kN is applied at the centre of spring whose permissible stress is 300N/mm2.The spring has a ratio of total depth to width of about 2.5. Determine the width, thickness, deflection and length of all leaves. | CO3 | 16 |
| 5. | a. | Distinguish between shaft and axle with examples. | CO1 | 2 |
|  | b. | Design a clamp coupling to transmit 30 kW at 960 rpm. The allowable shear stress for shaft and key is 40 MPa. Crushing stress for key is 90 MPa. Number of bolts connecting the two halves are 6. Permissible tensile stress is 70 MPa. Assume coefficient of friction between the shaft and muff as 0.3. The design shear stress for coupling is 15 MPa. | CO3 | 18 |
| (OR) |  |  |  |  |
| 6. | a. | Design a rectangular key for a shaft of 70 mm diameter , 105 mm length and the shaft transmitting 35kW power . Shear stress, crushing stress for the key material are 42 MPa and 84 MPa. | CO2 | 4 |
|  | b. | A line shaft running at 1000rpm transmits 20 kW . It carries a central load of 1kN.It is supported in bearings, the distance between their centers being 1 meter. For permissible shear stress of 45 N/mm2 and bending stress of 70 N/mm2 , determine the diameter of shaft , assuming minor shock loading . | CO3 | 16 |
| 7. | a. | List the various stresses induced in the single eye and double eye of knuckle joint. | CO1 | 2 |
|  | b. | Mention few examples for permanent and temporary joints | CO1 | 2 |
|  | c. | Design a cotter joint to support a load of 6 kN. Permissible design stresses are: in tension 60 N / mm2, in crushing 90N / mm2, in shear 40 N/mm2. | CO3 | 16 |
| (OR) |  |  |  |  |
| 8. | a. | Write the advantages and disadvantages of welded and riveted joint. | CO1 | 2 |
|  | b. | What is the equation employed for the design of thick cylinders? | CO2 | 2 |
|  | c. | Design a knuckle joint for transmitting an axial load of 60 kN for the following stresses: in tension 60 MPa, in compression 75 MPa, in shear 40 MPa. Tabulate the specification. | CO3 | 16 |
|  |  | Compulsory: |  |  |
| 9. |  | A multi-cylinder engine is to run at a constant load with a speed of 600 rpm. On drawing the crank effort diagram to scale of 1 cm= 2500 N-m and 1 cm= 30°, the area above and below the mean torque line are in sq.cm as follows: +1.6, -1.72, +1.68, -1.91, +1.97, -1.62. The speed is to be kept within +1% of the mean speed of the engine. Find suitable dimensions of a cast iron flywheel rim assuming suitable proportions. | CO2 | 20 |