Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14ME2050 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ADVANCED INTERNAL COMBUSTION ENGINES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the stages of combustion in SI engine with the help of a p-θ diagram. | CO1 | 10 |
|  | b. | Derive the expression to calculate the air fuel ratio for a simple carburetor neglecting the compressibility of air. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the factors affecting the combustion process in SI engine. | CO1 | 12 |
|  | b. | Explain the design principles underlying a good combustion chamber of a spark ignition engine. | CO1 | 8 |
| 3. |  | Describe the phenomenon of knock in CI engines. Explain the factors which influence the knock in CI engines. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A gas engine working on otto cycle has cylinder bore of 200 mm and stroke length of 250 mm. The clearance volume is 1570 cm3. The pressure and temperature at the beginning of the compression are 1 bar and 27 °C respectively. The maximum temperature of the cycle is 1400 °C. Determine the pressure and temperature at the salient points, the air standard efficiency, the work done and the mean effective pressure. (for air take Cv= 0.718 kJ/kg K, and R= 0.287 kJ/kg K). Also calculate the ideal power developed by the engine, if the number of working cycles per minute is 500. | CO1 | 20 |
| 5. | a. | How is NOx is formed in the exhaust of IC engines? Explain the important engine variables that affect NOx emissions? | CO2 | 10 |
|  | b | What are the design and operating variables which may decrease the formation of HC in the exhaust of an SI engine? | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Describe the working principle of the NDIR gas analyser with the help of a schematic diagram. How is this analyzer calibrated? | CO2 | 12 |
|  | b. | What are the major pollutants from the exhaust of gasoline and diesel engines? What are the detrimental effects of these pollutants? | CO2 | 8 |
| 7. | a. | Describe the refining process of petroleum by the method of fractional distillation? | CO3 | 10 |
|  | b. | What are the advantages and disadvantages of using gaseous fuels in IC engines? | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Briefly explain non petroleum based liquid fuels used in IC engines? | CO3 | 14 |
|  | b. | What are the fuel characteristics on which the performance of SI engine depends? | CO3 | 6 |
|  |  | Compulsory: |  |  |
| 9. |  | Describe the effect of volatility on the performance of SI engine? | CO3 | 20 |