Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2051 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # REFRIGERATION AND AIRCONDITIONING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A simple ammonia vapour compression system is operating between the pressure limits of 12 bar and 2.5 bar. The liquid is sub-cooled to 20ºC and the temperature of the vapour leaving the compressor is 100ºC. Plot the processes on a temperature-entropy, pressure-enthalpy diagram and determine the coefficient of performance of the system. | CO1 | 10 |
|  | b. | Explain Boot – Strap air cooling system with neat sketches. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | A dense air refrigeration cycle operates between pressure of 4 bar and 16 bar.The air temperature after heat rejection to surroundings is 37° C and the air temperature at exit of refrigerator is 7° C.The isentropic efficiencies of turbine and compressor are 0.85 and 0.8 respectively. Determine the compressor and turbine work per TR,C.O.P and power per TR.Take γ=1.4 and cp=1.005 kJ/kgK | CO1 | 20 |
| 3. |  | Describe the properties of various inorganic refrigerants. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Write short notes on any four types of evaporators. | CO2 | 20 |
| 5. |  | The humidity ratio of atmospheric air at 28ºC dry bulb temperature and 760 mm of mercury is 0.016 kg/kg of dry air. Determine the partial pressure of water vapour, relative humidity, dew point temperature, specific enthalpy and vapour density using empirical relations and steam tables. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | List the classification of air- conditioning system and discuss the factors afftecting comfort air- conditioning system. | CO4 | 20 |
| 7. | a. | An air-conditioning plant is required to supply 60 m3 of air per minute at 21ºC dry bulb temperature and 55% RH. The outside air is at 28ºC DBT and 60% RH. Determine the mass of water drained and the capacity of the cooling coil. Assume that the air-conditioning plant first dehumidify and then cools the air. | CO5 | 10 |
|  | b. | State the factors that affect the optimum effective temperature. | CO5 | 10 |
| (OR) |  |  |  |  |
| 8. |  | A hall is to be maintained at 24ºC DBT and 60% RH under the following conditions: Outside design conditions = 38ºC DBT, 28ºC WBT Sensible heat load in the room = 46.4 kW Latent heat load in the room = 11.6 kW Infiltrated air = 1200 m3/h Apparatus dew point temperature = 10ºC Percentage of recirculated air from the hall = 60% If the recirculated air is mixed with the conditioned air after the cooling coil, find the condition of air leaving the cooling coil, condition of air before entering the hall, quantity of air entering the cooling coil, quantity of air passing through the hall and the refrigeration load on the cooling coil in tonnes. | CO4 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the applications of air-conditioning system in public buildings. | CO5 | 10 |
|  | b. | Explain how refrigeration is employed in milk chilling plant. | CO5 | 10 |