Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14MT2029 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STUDIO ACOUSTICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | A seat in a performance auditorium is 26m from the guitarist on stage. When the guitarist strikes a single note the sound pressure level of the direct sound of the note at the seat is measured to be 55dB. The first reflection from the nearest side wall arrives at the seat 105msec after the arrival of the direct sound. - How far does the reflection travel to reach the seat? - What is the SPL of the reflection at the seat, assuming perfect reflection at wall? - How long will the reflection be delayed after the arrival of the direct sound at the seat? | CO3 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Discuss upon Loudness perception and Directional perception of human ear. Pictorially represent the different parts of human ear and explain their functions. | CO1 | 20 |
| 3. |  | Calculate the average RT60 for a room with the given specifications: Dimensions: 20ft X 15ft X 10ft Treatment: Ceiling – Acoustic Tiles Flooring – wood Two adjacent walls – Gypsum board: ½ inch Other walls – Concrete block, coarse The Absorption coefficients are given for each material as below: Material; 125Hz; 250Hz; 500Hz; 1kHz; 2kHz; 4kHz. Gypsum Board: 1/2inch; 0.29; 0.10; 0.05; 0.04; 0.07; 0.09. Wood; 0.15; 0.11; 0.10; 0.07; 0.06; 0.07. Acoustic Tiles; 0.07; 0.21; 0.66; 0.75; 0.62; 0.49. Concrete block, coarse; 0.36; 0.44; 0.31; 0.29; 0.39; 0.25. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Discuss upon Tone Burst Method and Reverberation Chamber method of calculating Absorption coefficient of a material. | CO1 | 12 |
|  | b. | Pictorially the difference in Diffraction of sound waves in the below given cases: | CO1 | 8 |
| 5. |  | Calculate the lowest 10 modes (Axial, Tangential and Oblique) of a room with a dimension length, width and height of 25ft, 16ft and 10ft respectively. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | For the below mentioned absorbtion materials briefly describe their absorption characteristics: - Glass fiber - Helmholtz Resonator - Bass Traps - Panel Absorbers - Drapes | CO1 | 20 |
| 7. |  | Design a Schroeder’s one dimensional diffusers with prime number 13. Draw the elevation of the prime number quadratic residue based well depths and mention the frequency range that would be diffused by the panel if the dimensions of the depths of 1 unit would 1m. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss the steps to be taken to isolate Walls, Ceiling, Floors and Windows of a Recording Studio. | CO3 | 16 |
|  | b. | Calculate the NRC (Noise Reduction Coefficient) value for the acoustic panel with the following absorption coefficient: Material; 125Hz; 250Hz; 500Hz; 1kHz; 2kHz; 4kHz. Carpet; 0.08; 0.24; 0.57; 0.69; 0.71; 0.73. | CO1 | 4 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss acoustic treatment that need to be done for a Listening room. Discuss about the Loudspeaker placement and Low and Mid frequency considerations that need to be taken into context. | CO3 | 20 |