Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14PH2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # VACUUM AND THIN FILM TECHNOLOGY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | a. | ___________ pumps use no oil and operates like jet engines. | CO1 | 1 |  |  |  |  |  |  |
|  | b. | ___________ is a device that reduces the pressure of gas in a container. | CO1 | 1 |  |  |  |  |  |  |
|  | c. | Mention the principle of working of cryo pump. | CO1 | 3 |  |  |  |  |  |  |
|  | d. | Explain with suitable diagram the principle, instrumentation and operation turbo molecular of pump. | CO1 | 15 |  |  |  |  |  |  |
| (OR) |  |  |  |  |  |  |  |  |  | (OR) |
| 2. | a. | ___________ pump is based on the principle of displacement of gap. | CO1 | 1 |  |  |  |  |  |  |
|  | b. | ___________ is a vacuum pump that traps gases and vapors by condensing them on a cold surface. | CO1 | 1 |  |  |  |  |  |  |
|  | c. | Discuss the importance of vacuum coating in thin film technology. | CO1 | 3 |  |  |  |  |  |  |
|  | d. | Explain with neat diagram, the pumping mechanism and working of diffusion pumps. | CO1 | 15 |  |  |  |  |  |  |
| 3. | a. | ___________ gauge is also called ionization gauge. | CO1 | 1 |  |  |  |  |  |  |
|  | b. | ___________ gauges incorporate the principle of energy transfer between a hot wire and a room temperature gauge wall. | CO1 | 1 |  |  |  |  |  |  |
|  | c. | Differentiate viscous and molecular flow regimes. | CO1 | 3 |  |  |  |  |  |  |
|  | d. | With suitable sketch, explain in detail, the construction and working of cold cathode gauge. Mention its advantages and disadvantages. | CO1 | 15 |  |  |  |  |  |  |
| (OR) |  |  |  |  |  |  |  |  |  | (OR) |
| 4. | a. | The ___________ gauge operates on the principle of momentum transfer. | CO1 | 1 |  |  |  |  |  |  |
|  | b. | ___________ gauges report pressure by measuring a pressure-dependent property of the gas. | CO1 | 1 |  |  |  |  |  |  |
|  | c. | Differentiate direct reading gauges and indirect reading gauges. | CO1 | 3 |  |  |  |  |  |  |
|  | d. | Discuss in detail, the construction and working of Capacitance Diaphragm gauge. | CO1 | 15 |  |  |  |  |  |  |
| 5. | a. | Insulators cannot be deposited by ___________ sputtering. | CO1 | 1 |  |  |  |  |  |  |
|  | b. | The type of adsorption due to the formation of chemical bond is called ___________. | CO1 | 1 |  |  |  |  |  |  |
|  | c. | Explain the principle of chemical vapor deposition. | CO1 | 3 |  |  |  |  |  |  |
|  | d. | Define interface. With a neat sketch explain the various interfacial processes. | CO2 | 15 |  |  |  |  |  |  |
| (OR) |  |  |  |  |  |  |  |  |  | (OR) |
| 6. | a. | Evaporation boat is made up of ___________ materials. | CO1 | 1 |  |  |  |  |  |  |
|  | b. | In vacuum evaporation the vapour stricking the surface per unit area at particular time is called ___________. | CO1 | 1 |  |  |  |  |  |  |
|  | c. | Mention the steps involved in sol-gel deposition. | CO1 | 3 |  |  |  |  |  |  |
|  | d. | Discuss the principle and working of RF sputtering technique in the deposition of thin film. Mention its advantages and disadvantages. | CO1 | 15 |  |  |  |  |  |  |
| 7. | a. | NEMS is the acronym for -----------------------. | CO2 | 1 |  |  |  |  |  |  |
|  | b. | ___________ is light emission from any form of matter after the absorption of photons. | CO3 | 1 |  |  |  |  |  |  |
|  | c. | Define Bragg’s law. | CO3 | 3 |  |  |  |  |  |  |
|  | d. | Describe how the structure of a material and the particle size are determined by X- ray diffraction studies. | CO3 | 15 |  |  |  |  |  |  |
| (OR) |  |  |  |  |  |  | CO3 |  |  |  |
| 8. | a. | _______ coatings are used to avoid the incoming sunlight from being reflected back off the solar cell. | CO3 | 1 |  |  |  |  |  |  |
|  | b. | Debye Scherrer formula for particle size determination is given by D = _______________. | CO3 | 1 |  |  |  |  |  |  |
|  | c. | Define Hall effect. | CO2 | 3 |  |  |  |  |  |  |
|  | d. | Describe in detail the basic parts and working of UV-Visible spectrophotometer. Explain how to determine the transmittance and absorbance of thin films with suitable equations. | CO3 | 15 |  |  |  |  |  |  |
|  |  | Compulsory: |  |  |  |  |  |  |  |  |
| 9. | a. | Discuss in detail the different structures of thin film transistor and the various steps involved in fabrication of thin film transistors. | CO2 | 12 |  |  |  |  |  |  |
|  | b. | Briefly discuss the steps in fabrication of a thin film solar cell. | CO2 | 8 |  |  |  |  |  |  |