Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 15EI2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BIOSIGNAL CONDITIONING CIRCUITS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | An op amp has bias currents IB1 and IB2 as 400nA and 300nA. Calculate the input bias current of the op amp. | CO1 | 5 |
|  | b. | Illustrate the application of a bio amplifier that inverts the input signal. | CO2 | 5 |
|  | c. | Design an opmap circuit whose output voltage is given by V0=V1-V2+V3-V4. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A bio potential amplifier gives an output voltage without connecting any electrodes from the patient. Analyze this problem. | CO3 | 10 |
|  | b. | Suggest suitable compensations for the above problem. | CO2 | 10 |
| 3. | a. | Design a comparator circuit which will give a threshold of 6V. | CO1 | 3 |
|  | b. | Comment on practical differentiator. | CO2 | 10 |
|  | c. | List the characteristics of a good instrumentation amplifier. | CO1 | 7 |
| (OR) |  |  |  |  |
| 4. | a. | Design a filter that attenuates a frequency less than 2kHz. | CO3 | 10 |
|  | b. | Analyse the losses in ideal integrator and show how it is compensated in a practical integrator. | CO2 | 10 |
| 5. | a. | In a PPG signal acquisition, a week signal was detected at 0.2Hz to 5Hz. Suggest suitable filtering circuit that attenuates signals greater than 5 Hz. | CO3 | 10 |
|  | b. | Design a biosignal conditioning circuit which outputs spikes response for a frequency of 2khz. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. |  | An IC 555 chip has been used to construct a pulse generator. Typical pin connections with components are shown below. It is desired to generate a square pulse of 10 kHz. Evaluate values of RA and RB if the capacitor C has the value of 0.01 µF for the configuration chosen. If necessary you can suggest modification in the external configuration. | CO3 | 20 |
| 7. | a. | Describe in detail on analog phase detectors. | CO1 | 10 |
|  | b. | Explain in brief about Phase Locked Loop with relevant schematics. | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | With a neat diagram explain magnetically coupled medical isolation amplifier and its significance. | CO2 | 12 |
|  | b. | Comment on optical mode of isolation in bio measurement. | CO3 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | With a neat diagram explain the significance of sample and hold circuit in biosignal data acquisition systems. | CO2 | 8 |
|  | b. | Discuss on various electrical interface problems and safety standards in bio potential measurements. | CO3 | 12 |