Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # INTELLIGENT INSTRUMENTATION SYSTEMS | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # 15EI2037 | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain in detail about the software development tools used in real time system. | CO2 | 8 |
|  | b. | Summarize the various design process and metrics that is to be optimized in an embedded system. | CO2 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | Design an embedded system to acquire and transmit the data of the incoming pulse from the patient at the input port. Support your hardware design with a block diagram and the software development with a flow diagram. | CO3 | 10 |
|  | b. | Comment briefly on the following interface related to embedded systems. - Switch Interface - LED Interface | CO1 | 10 |
| 3. | a. | Describe the generation of pulse width modulation signals in software and hardware. | CO2 | 10 |
|  | b. | Design an embedded based data acquisition system to measure and display the body temperature from the patient. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Consider a system which can control and monitor the desired temperature. With neat sketch, Illustrate how control system is applied to perform the above task. | CO2 | 10 |
|  | b. | Illustrate the operation of successive approximation type ADC. | CO2 | 10 |
| 5. |  | Design a stop watch system which has four keys: start, stop, pause and continue. The output should display the time in minutes and seconds in the display unit. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | It is desired to set up a completely automated automobile industry in which conveyors are used to transfer the parts from one station to the other. The stepper motors are used as the drivers in the conveyors. Enumerate the various methods involved in automatic control of a stepper motor using a microcontroller. | CO2 | 15 |
|  | b. | Identify the various services of simulators in an embedded system application development. | CO2 | 5 |
| 7. | a. | Outline the concepts of - Memory management functions. - Event functions. | CO3 | 10 |
|  | b. | Exemplify the various RTOS task states. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | With neat flow diagram, explain how priority inversion problem is solved by real time kernel. | CO1 | 16 |
|  | b. | List the various services of real time kernel. | CO1 | 4 |
|  |  | Compulsory: |  |  |
| 9. |  | With neat sketch, outline the different methods of handling interrupt services in RTOS environment. | CO3 | 20 |