Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 16AE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | STRUCTURAL MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. 35000N | An axial pull of 35000N is acting on a bar consisting of three lengths as shown in fig. below. If the young’s modulus = 2.1×105 N/mm2 , determine Stresses in each section. The lengths of each section are L1=20cm,L2=25cm and L3=22cm d2=3cm d3=5cm 35000N d1=2cmmm cmcm | CO1 | 10 |
|  | b. | Find the Young’s modulus of a brass rod of diameter 25mm and of length 250mm which is subjected to a tensile load of 50kN when the extension of the rod is equal to 0.3mm. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Two vertical rods one of steel and the other of copper are each rigidly fixed at the top and 50 cm apart. Diameters and lengths of each rod are 2cm and 4m respectively. A cross bar fixed to the rods at the lower ends carries a load of 5000N such that the cross bar remains horizontal even after loading. Find the stress in each rod and the position of the load on the bar. Take E for steel=2×105 N/mm2 and E for copper=1×105 N/mm2. | CO1 | 20 |
| 3. | a. | Derive an expression for the stresses on an oblique plane of a rectangular body, when the body is subjected to two mutually perpendicular normal stresses. | CO1 | 10 |
|  | b. | - A bar of cross sectional area 6000 mm2 is subjected to a tensile load of 50 kN applied at each end. Determine the normal stress on a plane inclined at 30º to the direction of loading. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Draw shear force and bending for the simply supported beam given below | CO2 | 20 |
| 5. | a. | A Cantilever beam 1.5 m long is loaded with a uniformly distributed load of 2kN/m run over a length of 1.25 m from the free end. It also carries a point load of 3kN at a distance of 0.25 m from the free end. Draw the shear force and bending moment diagrams for the Cantilever beam. | CO2 | 10 |
|  | b. | A cantilever of length 2m carries a uniformly distributed load of 2kN/m length over the whole length and a point load of 3kN at the free end. Draw the shear force and bending moment diagrams for the Cantilever beam. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Derive the expression of bending stress with the assumptions of simple bending. | CO2 | 10 |
|  | b. | Calculate the maximum stress induced in a cast iron pipe of external diameter 40mm,off internal diameter 20mm and of length 4m when the pipe is supported at its ends and carries a point load of 80N at its centre. | CO2 | 10 |
| 7. | a. | Find the expression for the slope at the supports of a simply supported beam carrying a point load at the centre. | CO4 | 10 |
|  | b. | Find the expression of deflection for a cantilever beam carrying a point load at the free end. | CO4 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A closely coiled helical spring is to carry a load of 500N.Its mean coil diameter is to be 10 times that of the wire diameter. Calculate these diameters if the maximum shear stress in the material of the spring is to be 80 N/mm2. | CO3 | 10 |
|  | b. | A laminated spring 1m long is made up of plates each 5cm wide and 1cm thick. I the bending stress in the plate is limited to 100 N/mm2, how many plates would be required to enable the spring to carry a central point load of 2kN? If E== 2.1×105 N/mm2 ,what is the deflection under the load? | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Determine the diameter of a solid steel shaft which will transmit 90kW at 160 rpm. Also determine the length of the shaft if the twist must not exceed 1 degree over the entire length. The maximum shear stress is limited to 60 N/ mm2. Take the value of modulus of rigidity =8×104 N/ mm2. | CO3 | 20 |