Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 16AE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | STRUCTURAL MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A brass bar, having cross sectional area of 1000mm2,is subjected to axial forces as shown in fig.below. Find the total elongation of the bar.Take E=1.05×105N/mm2 | CO1 | 10 |
|  | b. | An axial pull of 40000N is acting on a bar consisting of three sections of length 30cm,25cm and 20cm and of diameters 2cm,4cm and 5cm respectively. If the Young’s modulus =2×105N/mm2, determine: - Stress in each section - Total extension of the bar | CO1 | 5 5 |
| (OR) |  |  |  |  |
| 2. |  | A steel rod of 3cm diameter is enclosed centrally in a hollow copper tube of external diameter 5cm and internal diameter of 4cm.The composite bar is then subjected to an axial pull of 45000N. If the length of each bar is equal to 15cm,determine: - The stresses in the rod and tube - Load carried by each bar Take E for steel=2.1×105 N/mm2 and for copper=1.1×105 N/mm2 | CO1 | 10 10 |
| 3. | a. | Derive an expression for the stresses on an oblique plane of a rectangular body, when the body is subjected to direct stresses in two mutually perpendicular directions. | CO1 | 10 |
|  | b. | At a certain point in a strained material, the intensities of stresses on two planes at right angles to each other are 20 N/mm2 and 10 N/mm2 both tensile. They are accompanied by a shear stress of magnitude 10 N/mm2.Find graphically or otherwise, the location of principal planes and evaluate the principal stresses. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | A cantilever 1.5m long is loaded with a uniformly distributed load of 2kN/m run over a length of 1.25m from the free end. It also carries a point load of 3kN at a distance of 0.25m from the free end. Draw shear force and bending moment diagrams of the cantilever. | CO2 | 10 |
|  | b. | Draw shear force and bending moment diagrams for a simply supported beam of length 8m carrying a uniformly distributed load of 10kN/m for a distance of 4m as shown in fig.below. RA RB | CO2 | 10 |
| 5. |  | Derive an expression for bending stress and also identify the factors affecting bending stress. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Derive an expression for the slope and deflection of a simply supported beam of length L, carrying a uniformly distributed load by moment area method. | CO3 | 10 |
|  | b. | A beam 4m long, simply supported at its ends, carries a point load W at its centre. If the slope at the ends of the beam is not to exceed 1ͦ, find the deflection at the centre of the beam. | CO3 | 10 |
| 7. |  | Derive an expression for the slope and deflection of a simply supported beam of length L, carrying an eccentric point load by Macaulay’s method. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | A laminated spring 1m long is made up of plates each 5cm wide and 1cm thick. If the bending stress in the plate is limited to 100N/mm2, how many plates would be required to enable the spring to carry a central point load of 2kN?If E=2.1×105 N/mm2,what is the deflection under the load? | CO3 | 10 |
|  | b. | A closed coil helical spring is to carry a load of 500N.Its mean coil diameter is to be 10 times that of the wire diameter. Calculate these diameters if the maximum shear stress in the material of the spring is to be 80 N/mm2. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Two shafts of the same material and of same lengths are subjected to the same torque, if the first shaft is of a solid circular section and the second shaft is of hollow circular section,whose internal diameter is 2/3 of the outside diameter and the maximum shear stress developed in each shaft is the same, compare the weights of the shafts. | CO3 | 20 |