Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 16AE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT STRUCTURES-I | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | A pin-jointed truss supported by a pinned support at A and a roller support at G carries three loads at joints C, D and E as shown in Figure 1. Determine the magnitude and sense of the forces induced in members using method of joint. Fig.1 | CO2 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Figure.2 shows a beam simply supported at the supports A and C and is continuous over the supports B. Assuming EI is constant draw the bending moment and shear force diagram using three moment equation. Fig.2 | CO2 | 20 |
| 3. |  | Derive the Euler’s equation for the Column with both ends are hinged or pinned and both ends are fixed. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A continuous beam ABCD is simply supported over three spans, such that AB= 8m, BC=12m and CD = 5m. It carries uniformly distributed load of 4 kN/m in span AB, 3 kN/m in span BC and 6 kN/m in span CD. Draw the shear force and Bending moment diagram using moment distribution method. | CO2 | 20 |
| 5. | a | A beam having the cross-section shown in Fig.3 is subjected to a bending momentof 1500 Nm in a vertical plane (Mx = 1500 Nm). Calculate the maximum direct stress due to bendingstating the point at which it acts.(Use Neutral axis Method) Fig.3 | CO2 | 10 |
|  | b. | Derive equation of moment of inertia with about principle axes. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | A box beam shown in fig.4subjected to bending moment MX = 100 Nm and MY = 50 Nm. Boom areas A1 = 6 x10-4 m2, A2 = 5 x10-4 m2, A3 = A4 = 4 x10-4 m2, Calculate the bending stress of booms. (Use principle axes Method) Fig.4 | CO1 | 10 |
|  | b. | Derive equation of normal bending stress with respect to neutral axis. | CO2 | 10 |
| 7. |  | Using the method of virtual work, find the vertical deflection component of point E of the truss shown in fig.5. Cross-sectional areas of members are: AE and FD = 250 mm2 ; EF and EC = 1875 mm2 ; AB, BC, CD, EB and FC =1250 mm2 ; young’s modulus E = 200 kN/mm2 . Fig.5 | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Briefly explain the different type of material used in Aircraft construction. | CO2 | 10 |
|  | b. | A solid round bar 60mm in diameter and 2.5 m long is used as a strut. One end of the strut is fixed, while its other end is hinged. Find the safe compressive load for this strut, using Euler’s formula. E=200 GPa and factor of safety = 3. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Briefly explain the different types of mechanical vibrations and failures of aircraft structures due to vibration. | CO3 | 20 |