Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 16AE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT STRUCTURES-I | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Determine resultant force of the each member of truss shown in fig.1. Fig.1 | CO2 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Calculate the vertical deflection of the Joint B and the horizontal movement of the support D in the truss shown in fig. 2. The cross-sectional area of each member is 1800mm2 and Young’s modulus, E, for materials of the members is 200 000 N/mm2 . Fig.2 | CO2 | 20 |
| 3. |  | A continuous beam ABC consists of two consecutive spans AB and BC 4 meters each and carries a distributed load of 60 kN per meter run. The end A is fixed and the end B and C are simply supported. Find support moments, support reactions and draw the B.M. and S.F. Diagram using three moment equation. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A continuous beam ABC is simply supported over two spans, such that AB=6 and BC=4m. Span AB carries a point load 3kN at the distance of 2m from A. Span BC carries uniformly distributed load of 1kN/m. Draw the bending moment and shear stress diagram using moment distribution method. | CO2 | 20 |
| 5. | a | Derive the Euler’s equation for the Column with one end fixed and other end free. | CO2 | 10 |
|  | b. | Calculate the safe compressive load on a hollow cast iron column (one end fixed and other hinged) of 150 mm external diameter, 100 mm internal diameter and 10 m length. Use Euler’s formula with a factor of safety of 5 and E= 95 GN/m2. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Derive equation of moment of inertia, normal bending stress about principle axes. | CO1 | 10 |
|  | b. | Briefly explain the different type of composite materials used in Aircraft construction. | CO2 | 10 |
| 7. |  | The cross-section of a beam has the dimensions shown in fig.3. If the beam is subjected to a negative bending moment of 100 kN m applied in a vertical plane (i.e. MX = -100 kN), Calculate the maximum direct stress in the stating clearly the point at which it acts.( Neutral axis method) Fig.3 | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A beam having the cross-section shown in Fig.4 is subjected to a bending moment of 1500 Nm in a vertical plane (Mx = 1500 Nm). Calculate the maximum direct stress due to bending stating the point at which it acts.(Use principle axes Method) Fig.4 | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Briefly explain the different types of mechanical vibrations and failures of aircraft structures due to vibration. | CO3 | 20 |