Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 16AE2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | AIRCRAFT STRUCTURES-II | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Questions | Course Outcome |  |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | Find the shear flow distribution in a thin walled Z section whose thickness is t, height h, flange width h/2 and subjected to a vertical shear load through its shear center. | CO1 |  |  | 20 |
| (OR) |  |  |  |  |  |
| 2. | Calculate the shear ﬂow distribution in the channel section shown in fig.2 produced by a vertical shear load of 4.8 kN acting through its shear centre. Assume that the walls of the section are only effective in resisting shear stresses while the booms, each of area 300 mm2, carry all the direct stresses. Fig.2 | CO1 |  |  | 20 |
| 3. | Find the shear flow for the closed section shown in fig.3. Subjected to a vertical force V passing through the shear center. Thickness t for the entire wall. Fig.3 | CO1 |  |  | 20 |
| (OR) |  |  |  |  |  |
| 4. | # The thin-walled single cell beam shown in Fig.4 has been idealized into a combination of direct stress carrying booms and shear stress only carrying walls. If the section supports a vertical shear load of 10 kN acting in a vertical plane through booms 3 and 6, calculate the distribution of shear ﬂow around the section. Boom areas: B1=B8=200 mm2, B2=B7=250 mm2, B3=B6=400 mm2, B4 = B5 =100 mm2. # Fig.4 | CO1 |  |  | 20 |
| 5. | Find the shear flow and twist per unit length of the fig.5 shown. Take G = 25x105 N/cm2 and radius R=10cm. Fig.5 | CO1 |  |  | 20 |
| (OR) |  |  |  |  |  |
| 6. a. | Explain the Gerard method of estimating crippling stress. | CO2 |  |  | 10 |
| b. | An angle section column has cross-section dimensions 4cm x 3 cm x 2 mm.Determine the crippling load using Needham Method. E= 200GPa, Fcy =800 KPa. | CO2 |  |  | 10 |
| 7. | - Find the margin of safety in buckling for the box beam shown in fig.6. Given P1 = P2 =10kN. Area of the each stringer = 2 cm2 and the sheet thickness is 1.5mm throughout. Assume the sheets are effective in bending and made of 2024-T3 Aluminum alloy. For a/b= 3, kc = 4, ks= 5.8. Fig.6 | CO2 |  |  | 20 |
| (OR) |  |  |  |  |  |
| 8. | Calculate the distribution of stiffener loads and the shear flow distribution in the web panels shown in fig.7. Assuming that the latter are effective only in shear. Fig.7 | CO3 |  |  | 20 |
| Compulsory: |  |  |  |  |  |
| 9. a. | Explain the principle of semi-Tension field beam and find an expression for Web stiffener load and Flange axial load | CO3 |  |  | 12 |
| b. | Write the short notes on the idealization and analysis of aircraft wing structure and fuselage. | CO3 |  |  | 8 |