Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 16EC2004 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRON DEVICES AND CIRCUITS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain in detail the operation of a PN junction Diode with appropriate circuit diagram, graph and support the same with the shockleys equation. | CO1 | 15 |
|  | b. | Determine VD, VR and ID for the series diode configuration shown below. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Compare Zener breakdown to an avalanche breakdown. | CO1 | 10 |
|  | b. | In the circuit shown below the zener diode is ideal and the zener voltage is 6V. Find the output voltage V0 in volts. | CO1 | 10 |
| 3. |  | Draw and explain the operation of a CE configuration.Bipolar Junction transistor with its input and output characteristic curves. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Explain in detail the Voltage divider biasing and comment on its stability. | CO1 | 10 |
|  | b. | In the circuit shown below, assume that the transistor is in active region. It has a large beta and its base emitter voltage is 0.7 V.Find IC | CO2 | 10 |
| 5. |  | Describe the enhancement Type MOSFET operation with a detailed Transfer curve and schokleys equation. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Represent the circuit given below in its AC equivalent model, Using re model for the transistor also List out the steps involved in DC analysis and AC analysis. | CO2 | 10 |
|  | b. | State the roles of the capacitors shown in the figure below | CO2 | 10 |
| 7. | a. | In the frequency response curve of a RC coupled Amplifier detail out the reasons for - Drop in gain in lower frequencies. - Constant Gain in Mid frequencies - Drop in Gain in Higher Frequencies | CO2 | 15 |
|  | b. | Determine the Voltage, Current and Power Gain of an amplifier that has an input signal of 1mA at 10mV and a corresponding output signal of 10mA at 1V. Also, express all three gains in decibels, (dB) | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Give the block diagram representation of Voltage Shunt feedback amplifier and derive the expression of input impedance, output impedance and gain of the amplifier. | CO3 | 10 |
|  | b. | Derive the Maximum Efficiency of a Class B Amplifier and compare the result with Class A Amplifier. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | List the three requirements for proper oscillator operation. | CO3 | 3 |
|  | b. | For a transistorized RC oscillator, select the value of capacitor C in order to provide 2 KHz oscillator frequency with resistance Rc=10KOhms , R=8KOhms. | CO3 | 5 |
|  | c. | State the Barkhausen Criterion and the results of not fulfilling this criterion. | CO3 | 5 |
|  | d. | With a neat circuit diagram explain the operation of RC Phase shift oscillator. | CO3 | 7 |