Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 16EI2004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AUTOMOTIVE ELECTRONICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | With suitable supporting sketches explain the operation of oxygen sensor. Investigate on the improvements caused by these sensors on vehicle performance. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Integrate various modules for electronically monitoring / controlling the various systems in a vehicle for achieving better performance. | CO3 | 20 |
| 3. |  | Discuss with relevant sketch about the functioning of MAP and MAF sensors during measuring air flow | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Elaborate the required characteristics of the motor and its construction when it is to be used for cranking the engine. | CO2 | 20 |
| 5. |  | Discuss in detail about the following engine control modes with respect to A/F ratio, open / closed loop operation - Engine Cranking - Engine Warmup - Engine Idling - Acceleration Enrichment - Deceleration Leaning | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Develop a braking mechanism with electronic control which avoids skidding of vehicle when applying brake. Also investigate about how these systems reduce the potential of accidents | CO3 | 20 |
| 7. |  | Compare the conventional ignition system with the solid state ignition system and with diagram explain about the distributor less ignition system | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Investigate interms of application, specification and other features of the various automotive communication protocols like LIN, CAN, MOST and Flexray | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Infer the various effects on maintaining low tire pressure. Develop a system to address the same. Provide suitable block diagrams to explain the operation. | CO3 | 20 |