Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 16ME2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HEAT AND MASS TRANSFER | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | State Fourier law of heat conduction and by using it drive an expression for steady state heat conduction through a plane wall of thickness L maintains its two surfaces at temperatures T1 and T2, respectively. | CO1 | 5 |
|  | b. | A furnace wall is made of composite wall of total thickness 550mm. The inside layer is made of refractory material (k=2.3 W/mK) and outside layer is made of an insulating material (k=0.2 W/mK). The mean temperature of the glass inside the furnace is 9000C and interface temperature is 5200C. The heat transfer coefficient between the gases and inner surface can be taken as 230 W/m2 0C and between the outside surface and atmosphere as 46 W/m20C. Taking air temperature = 300C, calculate : (i) Required thickness of each layer, ii) The rate of heat loss per m2 area, and iii) The temperatures of surface exposed to gases and of surface exposed to atmosphere. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | A Steel tube(k=45 W/m.K) of outside diameter 7.6 cm, and thickness 1.3 cm, is covered with an insulating material (k=0.2 W/m.K) of thickness 2 cm. A hot gas at 3300C, with convection coefficient of 200 W/m2.K, is flowing inside the tube. The outer surface of the insulation is exposed to ambient air at 300C, with convection coefficient of 50 W/m2K. Calculate i) Heat loss to air from the 5 m long tube, ii) The temperature drop due to thermal resistance of the hot gases, steel tube, the insulation layer and the outside air. | CO1 | 15 |
|  | b. | A Solid sphere (k=39 W/mK) 10 cm in diameter generates heat at a uniform rate of 5x106 W/m3. The outer surface of sphere is exposed to an ambient at 500C with heat transfer coefficient of 400 W/m2K. Calculate the maximum temperature in the solid and its location. | CO1 | 5 |
| 3. | a. | Calculate the steady state temperatures at points 1,2,3 and 4 using the numerical method 1000C 1000C 1000C 5000C 1 2 4 3 | CO1 | 5 |
|  | b. | An egg with mean diameter of 4 cm is initially at 250C. It is placed in boiling water for 4 min and found to be consumer’s taste. For how long should be a similar egg for same consumer be boiled when taken from refrigerator at 20C. Use lumped system analysis and take thermophysical properties of egg as k=12 W/m.K, h=125 W/m2.K, C=2000 J/kg.K, =1250 kg/m3 | CO1 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Explain the significance of Heisler and Grober charts in solving transient conduction problems. | CO1 | 5 |
|  | b. | Ice ball of 10 mm diameter at -320C are exposed to an air current at 150C with a convection heat transfer coefficient of 200 W/m2K. Determine the time when the surface layer will began to melt. Also determine the center, mid surface temperatures and heat removed. Use the following property values, density=920kg/m3, specific heat= 2040 J/kgK. Thermal conductivity =2 W/mK. | CO1 | 15 |
| 5. | a. | State Buckingham’s  theorem. Explain the various parameters used in free convection. Using dimensional analysis obtain an expression for Nusselt number in terms of Grashof and Prandtl numbers. | CO1 | 15 |
|  | b. | A very long fin has 25mm diameter, has one end maintained at 1300C.The surface of the fin is exposed to ambient air at 300C. The thermal conductivity of the fin material is 400 W/mK. If the convective heat transfer coefficient is 40 W/m2K, find the heat loss from the fin. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Explain velocity developing region and hydrodynamic entry length for flow in a circular tube. | CO1 | 5 |
|  | b. | Air at 200C and atmospheric pressure is flowing over a flat plate at a velocity of 3 m/s. If the plate is 30cm wide and at a temperature of 600C, Caalculate at x=0.3 m; i) thickness of velocity and thermal boundary layers, ii) local and average friction coeficients, iii) Local and average heat transfer coefficients, iv) Total drag force on the plate. | CO1 | 15 |
| 7. | a. | Expalin the term fouling factor. | CO2 | 5 |
|  | b. | A chemical having specific heat of 3.3 kJ/kgoK flowing at the rate of 20000 kg/h enters a parallel flow heat exchanger at 120oC. The flow rate of cooling water is 50000 kg/h with an inlet temperature of 20oC. The heat transfer area is 10m2 and the overall heat transfer coefficient is 1050 W/m2 oK. Find i) the effectiveness of the heat exchangers ii) the outlet temperature of water and chemical. | CO2 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Explain Fick’s law of diffusion. What is mass diffusivity? What is its dimension? | CO3 | 5 |
|  | b. | Two large tanks maintained at the same temperature and pressure are connected by a circular 0.15m diameter direct which is 3m in length one tank contains a uniform mixture of 60 mole % ammonia and 40 mole % air and the other tank contains a uniform mixture of 20 mole % ammonia and 80 mole % air. The system is at 273 K and 1.013 X 105 pa. Determine the rate of ammonia transfer between the two tanks. Assuming a steady state mass transfer. | CO3 | 15 |
|  |  | Compulsory: |  |  |
| 9. | a. | In a double pipe heat exchanger hot fluid with a specific heat of 2300 J/kg K enters at 380C and leaves at 300C. Cold fluid enters at 25C and leaves at 210C. Calculate the heat exchanger area required for counter flow. Take overall heat transfer coefficient is 750 W/m2 K and mass flow rate of hot fluid is 1 kg/s. | CO2 | 10 |
|  | b. | Write the classification of Heat exchangers with neat sketches | CO1 | 5 |
|  | c. | What is Radiation? How does thermal radiation differ from other types of electromagnetic radiation? | CO1 | 5 |