Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17AE3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED COMPUTATIONAL FLUID DYNAMICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Formulate the differential form of Navier-Stokes equation in Cartesian coordinates. | CO1 | 10 |
|  | b. | Discuss the different types of Partial Differential Equations used in the analysis of fluid flows. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Derive the Continuity equation in three-dimensional Cartesian coordinate system for an incompressible flow. | CO1 | 10 |
|  | b. | Appraise the applicability of hyperbolic partial differential equations with an example. | CO1 | 10 |
| 3. |  | Formulate the finite-difference representation of a second-order derivative to second-order accuracy. | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Construct the finite-volume representation of the equation governing the diffusion of temperature in two-dimensional coordinate system with a clear description of the nodes used. | CO3 | 20 |
| 5. |  | A solid bar of length 1.5 m is maintained at 250oC at the left side and 35oC at the right side. Compute the temperature distribution in the solid bar by formulating the finite-difference algebraic equation and evaluate it using the Tridiagonal Matrix Algorithm. Use a maximum of four grid points. Assume one-dimensional heat diffusion in the solid bar. | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Solve the following system of equations using Gauss-Seidel method without relaxation. | CO4 | 20 |
| 7. |  | Transform the following equation defined in the physical plane to the corresponding equation in the computational plane. | CO5 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Discuss the application of adaptive grids with respect to the computation of the boundary layer flow field over a flat plate. | CO5 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Justify the necessity of using turbulence models in simulating turbulent flows by supplementing it with a short discussion on the Mixing Length model. | CO6 | 20 |