Reg.No. ____________

End Semester Examination – April/May 2018

| # Code : | # 17CE3002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STRUCTURAL DYNAMICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Develop the effective stiffness of the combined spring system shown in figure and write the equation of motion for the spring mass system. k k x(t) M F(t) k | CO1 | 6 |
|  | b. | A spring mass system has a stiffness of “K” N/m and mass of “M” kg. It has natural frequency of vibration of 12Hz. If an extra 2kg is coupled to “M” the frequency reduces to 11Hz. Determine K and M. | CO3 | 14 |
| (or) |  |  |  |  |
| 2. | a. | Using Duhamel’s Integral find the response with zero initial conditions for the forced vibration phase of the SDOF system shown | CO4 | 10 |
|  | b. | A vibrating system consisting of a weight of 1200 kN and a spring stiffness of 100 kN/m is viscously damped so that the ratio of two consecutive amplitudes is 1 to 0.80. determine a) logarithmic decrement, b) natural frequency, c)damping ratio, d) damping coefficient and e) damped natural frequency | CO3 | 10 |
| 3. | a. | For a two storey shear building with the following data determine i) the normalized modal shapes of vibration, ii) verify the orthogonality conditions between the modes. m1= 20000 kg, m2 = 11500 kg, a11= 1.00, a21= 1.263, a12 = 1.00 a22 = -1.629. | CO2 | 10 |
|  | b. | A two storey building having a floor weight 1200kN, 600kN, for the first and second floor respectively. The height of each floor is 3m and EI = 30x1012 Nmm2 . Analyze the natural frequencies and mode shapes. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Justify the statement “ normal modes are orthogonal”. | CO2 | 10 |
|  | b. | Determine the eigen value and eigen vector of the two degree of freedom system shown in figure K 1.5m k m | CO3 | 10 |
| 5. |  | A three storey building has seismic weights of 200 kN, 300 kN and 420 kN at I, II and III storeys respectively; The corresponding stiffnesses are 20000 kN/m, 25000 kN/m and 30000 kN/m. (i) Determine the model frequencies. (ii) Sketch the mode shapes. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Determine the natural frequency and mode shape of the system shown in figure using transfer matrix method | CO5 | 20 |
| 7. | a. | Find out the solution for equation of motion and find out the natural frequency and mode shapes with one end fixed and other end simply supported. | CO4 | 10 |
|  | b. | Determine the first two frequencies of a Cantilever beam by Rayleigh-Ritz method by assuming | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Formulate the equation of motion by conservation of energy method. | CO1 | 10 |
|  | b. | Derive the equation of motion for flexural beam subjected to forced vibration. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Using Mode superposition method , find the response of a two degree of freedom system whose Mass and Stiffness Matrices are given by the following. and The forcing function Sin 10 t. The system starts at rest. | CO6 | 12 |
|  | b. | Evaluate the algorithm for step by step solution of linear system using Wilson θ method. | CO6 | 8 |