Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # APPLIED ELASTICITY AND PLASTICITY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Differentiate between body forces and surface forces with examples. | CO1 | 2 |
|  | b. | Explain the principle of Super Position. | CO1 | 2 |
|  | c. | Develop the differential equations of equilibrium in 3D Cartesian co-ordinates. | CO2 | 4 |
|  | d. | Develop the strain displacement relations in rectangular coordinates. | CO2 | 4 |
|  | e. | The stress tensor at a point is defined by array as shown below. Determine the stress invariants, principal stresses and principal directions. σ=358510802 | CO3 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the concept of stress at a point. | CO1 | 2 |
|  | b. | List the properties of Direction Cosines. | CO1 | 2 |
|  | c. | Develop the stress transformation law in 3D rectangular coordinates. | CO2 | 6 |
|  | d. | Develop the Generalized Hook’s law for an isotropic material. | CO2 | 4 |
|  | e. | The strain field in a body is given by Ԑxx = 0.005z γxy = 0.003xy Ԑyy = 0.001x γyz = -0.001xz Ԑzz = -0.002xy γzx = 0.001y Check whether it is a compatible strain field. | CO3 | 6 |
| 3. | a. | Show that the following function is a stress function for a plane stress problem and determine the stresses. f = (q/8c3)[x2(y3-3c2y+2 c3)-(1/5)( y3(y2-2c2))] | CO4 | 4 |
|  | b. | Determine the stresses in an infinite plate with hole subjected to a tension. | CO4 | 8 |
|  | c. | Prove the following stress compatibility equations for plane strain problem. ▼2 ( σx + σy) = −1(1+y)δFxδx+δFyδy | CO2 | 6 |
|  | d | Explain plane strain problem with examples. | CO2 | 2 |
| (OR) |  |  |  |  |
| 4. | a. | Investigate what problem of plane stress is solved by the stress function, φ=−Fd3xy2(3d−2y) applied to the region bounded in y = 0, y = d, x = 0 on the sidex+ve. | CO4 | 8 |
|  | b. | Determine the stresses in a rotating circular disc with a hole. | CO4 | 6 |
|  | c. | For a plane stress problem, develop G∇2u+E2(1−γ)∂∂x∂u∂x+∂v∂y+Bx=0 and a companion equation. | CO4 | 6 |
| 5. | a. | Why does Coulomb’s equation for torsion give erroneous result for non-circular section? | CO2 | 2 |
|  | b. | Develop the governing differential equation and the associated boundary condition for torsion of noncircular section by using St.Venant warping function approach. | CO5 | 10 |
|  | c | Develop the expression for maximum shear stress and unit angle of twist for an equilateral triangular shaft subjected to torque. | CO5 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | Explain membrane analogy for torsion. | CO2 | 2 |
|  | b. | An elliptical bar is subjected to a twisting moment T. Derive the expressions for shear stresses and angle of twist at any point in the bar and hence the maximum shear stress. | CO5 | 8 |
|  | c. | Develop the governing differential equation and the associated boundary condition for torsion of noncircular section by using Prandtl Stress function approach. | CO6 | 10 |
| 7. | a. | Develop the Governing equation for the analysis of beams on elastic foundation using Winkler model. | CO6 | 4 |
|  | b. | Develop the expressions for an infinite beam resting on elastic foundation for BM, SF, deflection and rotation if it is subjected to Point load. | CO6 | 10 |
|  | c. | An infinite beam on a Winkler foundation has the following properties. K = 0.3 kN/mm/mm. E=200GPa. A concentrated load of intensity 25 kN is applied to the beam. Compute the maximum deflection, shear force, bending moment and slope in the beam. The beam cross section is I shaped with flanges 100 X 10 mm and web 150 X 8mm. | CO6 | 6 |
| (OR) |  |  |  |  |
| 8 | a. | A semi-infinite beam (100 mm wide and 50 mm deep) resting on elastic foundation is hinged at one end and 12 kNm moment is applied at this end. Develop the necessary equations and hence determine the maximum deflection and stresses in the beam. E = 90GPa. Poisson’s ratio = 0.3. modulus of elastic foundation = 8.4 MPa. | CO6 | 10 |
|  | b. | Develop the necessary equations and hence determine the deflection of the finite beam on elastic foundation with one end built-in. udl= 250kN/m. L=4m. Modulus of foundation = 2MPa. EI=550X109 nmm2. | CO6 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | A rectangular beam 8 cm wide and 10 cm deep is 200 cm long and is simply supported at the ends. The yield strength for the beam material is 250 MPa. Determine the value of the concentrated load applied at the beam midspan if (i) the outermost fibers of the beam just start yielding, (ii) the outer shell upto 3 cm depth yields and (iii) whole of the beam yields. Assume linear idealized stress strain curve for the beam materials. | CO5 | 8 |
|  | b. | Distinguish nominal stress, true stress and mean stress. | CO5 | 2 |
|  | c. | Discuss Nadai’s Sand heap Analogy. | CO5 | 2 |
|  | d. | Develop the Prandtl Reuss Equations of plastic flow. | CO5 | 8 |