Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FINITE ELEMENT ANALYSIS OF STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define the Principle of Stationery potential energy. | CO1 | 2 |
|  | b. | Distinguish between Strong formulation and weak formulation. | CO2 | 2 |
|  | c. | Derive the shape functions for 4 noded axial element. | CO3 | 6 |
|  | d. | The following differential equation available for a physical phenomenon. d2 y +500 x2 = 0 0 ≤ x ≤ 1 dx2 By using the trial function y = a (x – x3) + b (x – x5), calculate the value of the parameters a and b by the Galerkin method of weighted residual. | CO4 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Define the Principle of virtual work and develop the stiffness matrix for a 2 noded axial element in local coordinates. | CO1 | 5 |
|  | b. | Using the variational principle, develop the equation of a beam and the associated boundary conditions from the expression for total potential energy. | CO3 | 5 |
|  | c. | Develop the stiffness matrix for a 2D beam element and anlyze a continuous beam of two equal spans 10 meter each carrying a udl 500kN/mter on the left span and 80kN/metre on the right span. The beam is fixed on the left end and on rollers at the right end. Breadth of the beam is 500mm and depth of the beam is 1000mmTake E = 2X105 N/mm2. | CO4 | 10 |
| 3. | a. | Evaluate the shape function N1, N2 and N3 at the interior P(3.85,4.8) for the planar triangular element shown | CO3 | 4 |
|  | b. | Develop the shape functions for LST in area coordinates and hence obtain the shape functions for a 4 noded planar triangular element. | CO3 | 6 |
|  | c. | Develop the shape functions for a 4 noded serendipity element and hence develop the stiffness matrix and consistent load vector for a iso parametric quadrilateral element. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Define uniqueness of mapping and indicate the significance of Jacobian determinant thereon. | CO1 | 5 |
|  | b. | Explain the theorems of iso parametric element. | CO2 | 5 |
|  | c. | Distinguish between Serendipity element, Lagrangean element and Hermitian element. | CO2 | 5 |
|  | d. | Develop the shape functions for a 8 noded serendipity element. | CO3 | 5 |
| 5. | a. | Using two point Gaussian quadrature function, evaluate the integral I=y=c=4y=d=6x=a= −2x=b=2(1−3x)2(4−y)5dxdy | CO6 | 10 |
|  | b. | Develop the shape functions and stiffness matrix and consistent load vector for a 8 noded brick element. | CO4 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | The four-node parent element abcd in the local ζ , η coordinate system is mapped into the derived element ABCD in the global x–y coordinate system, as shown in the Figure. Point p(0,–0.5) in the local coordinate system is mapped into point P in the global coordinate system. Obtain their rectangular coordinates and Jacobian determinant at the point P | CO5 | 10 |
|  | b. | Develop the shape functions and stiffness matrix for a 10 noded tetrahedran element. | CO4 | 10 |
| 7. | a. | Distinguish between C0, C1 and C2 plate bending elements. | CO2 | 4 |
|  | b. | Prove that a 12 dof rectangular plate element is not a conforming element. | CO5 | 8 |
|  | c. | Discuss the development of triangular plate bending element. | CO5 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss the concept of Degenerated Shell Element. | CO2 | 4 |
|  | b. | Explain the step by step procedure for analysis of a bridge deck by finite strip method. | CO5 | 8 |
|  | c. | Discuss in detail the 4 different approaches used to generate shell elements. | CO5 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | Develop the geometric stiffness matrix for a 2D truss element. | CO4 | 5 |
|  | b. | Develop the consistent mass matrix for a 2d Beam element. | CO4 | 5 |
|  | c. | Explain the various sources of solution errors in finite element analysis. | CO2 | 5 |
|  | d. | List the desirable features of FEA packages. | CO1 | 5 |