Reg.No. ____________

End Semester Examination – April/May 2018

| # Code : | # 17CE3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF BRIDGES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | How would you distinguish Class AA and 70 R loading? | CO1 | 3 |
|  | b. | When would you choose RCC T beam bridge? | CO2 | 3 |
|  | c. | A reinforced concrete simply supported slab is to be designed for a road bridge having the data given below: Width of carriageway - 7.5m Kerbs - 600mm wide Clear span - 6m Type of loading - IRC Class AA tracked vehicle Materials – M20 grade concrete, Fe415 grade tor steel Design the slab. | CO4 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | List all the conditions for adopting Courbon’s method. | CO1 | 3 |
|  | b. | How would you decide on the thickness of a culvert slab in the case of slab bridge and thickness of slab & depth of longitudinal girder in T beam bridges? | CO3 | 3 |
|  | c. | An RCC Tee beam and slab deck is to be designed for a major river crossing in a National Highway. The following data is available Clear width of roadway - 7.5m Wearing coat - 100mm No. of main girders - 3 Span (c/c of bearing) - 20m Spacing of cross girder - 5m c/c Loading - IRC Class AA tracked vehicle M20 grade of concrete and Fe 415 grade steel is used Design the deck slab using pigeud’s charts. | CO4 | 14 |
| 3. | a. | Suggest why box girders can be preferred for bridges. | CO3 | 5 |
|  | b. | Design a suitable section for the longitudinal girder of a post tensioned prestressed concrete T Beam Bridge for a National Highway crossing to suit the following data: Effective span - 15m Equivalent Live load - 20kN/m Adopt M45 grade of concrete with cube strength at transfer as 40 N/mm2. Loss ratio – 0.80 and the 7mm HTS wires initially stretched to 1200MPa.need to be used | CO4 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Specify the situation when the losses due to elastic shortening can be ignored in Prestressed concrete bridges. | CO3 | 5 |
|  | b. | Design a post- tensioned prestressed concrete slab bridge for a national highway crossing to suit the following data: Width of carriage way = 7.5m Kerbs = 600mm wide Clear Span = 8 m Type of loading = IRC Class AA or Class A whichever gives the worst effect Materials – M40 grade concrete and 7mm diameter high tensile wires with an ultimate tensile strength of 1500 N/mm2 housed in cables with 12 wires and anchored by Freyssinet anchorages of 150mm diameter. Compressive strength at transfer, fci = 35 N/mm2. Loss ratio = 0.8. Design the deck slab. | CO4 | 15 |
| 5. | a. | Can you list the role of stringers in railway steel bridges? | CO1 | 3 |
|  | b. | How would you define CDA? | CO3 | 3 |
|  | c. | Elaborate on the design procedure of a steel plate girder railway bridge. | CO4 | 14 |
| (OR) |  |  |  |  |
| 6. | a. | Sketch the cross sectional details of a top chord and bottom chord of a steel trussed girder bridge of large span. | CO4 | 5 |
|  | b. | A Pratt truss girder through bridge is provided for single broad gauge track. The effective span of bridge is 32m. The cross girders are spaced at 4m apart. The stringers are spaced 2m between centerlines. 0.60kN per meter stock rails and 0.40kN per metre checkrails are provided. Sleepers are spaced at 0.45m from center to center and are of size 2.8 m x 250 mm 250 mm. The weight of timber may be assumed as 7.5kN/m3. The main girders are provided at a spacing of 7m between their centerlines. Determine the forces in the central panel members | CO3 | 15 |
| 7. | a. | Can you list the different forces acting on an abutment? | CO5 | 5 |
|  | b. | Verify the stability of the abutment shown in Fig.1. The other salient details are given below: - Materials of the abutment - concrete - Density of the soil - 18 kN/m3 - Coefficient of friction - 0.5 - Angle of repose of the soil  = 30 Live load on the bridge: IRC Class AA (tracked) Span of the bridge = 12m. Angle of friction between the soil and concrete =  = 18. The bridge deck consists of three longitudinal girders 1.5m depth with a deck slab of 200mm thick. | CO5 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | How would you choose joints based on their functions? | CO2 | 5 |
|  | b. | Can you highlight the difficulties faced in incremental push launching method? | CO6 | 5 |
|  | c. | Enumerate the design procedure of well foundations in bridges | CO4 | 10 |
|  |  | Compulsory: |  |  |
| 9 |  | Enumerate a case study on a failure or construction of a major bridge | CO6 | 20 |