Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED STEEL STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Analyze the bending and shear capacity of beam of span 5.5m, carrying concentrated load of 50 kN at the center. Also check the web crippling and shear buckling capacity of the beam. Beam is laterally restrained. | CO4 | 15 |
|  | b. | Differentiate laterally restrained and laterally unrestrained beam? | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Design the laterally unrestrained beam of span 7m carrying transverse load of 20kN/m from the floor slab. Assume Fe410 steel. | CO4 | 15 |
|  | b. | How do you classify the steel section based on its local buckling capacity? | CO1 | 5 |
| 3. | a. | Design the braced column of length 3.5m carrying an axial compression load of 400kN and major axis bending of 100kNm in the frame. Evaluate the capacity of column under combined action of axial force and bending. | CO4 | 15 |
|  | b. | Write the possible failure modes of steel column? | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Design the bolted stiffened seat connection to carry the shear of 300kN and moment of 25kNm at the beam column joint. Assume the relevant design data. ISMB 350 as a Beam section ISHB 300 as a Column section | CO5 | 15 |
|  | b. | Discuss in detail the possibilities of failure of welded connection. | CO5 | 5 |
| 5. |  | Determine the loads (dead, live and wind) on the pitched roof truss for the following data Height of the column - 5m Span of the truss - 15m Height of the truss - 3m Spacing of the truss -4.5m Spacing b/w purlin - 2.5m c/c Assume the roof covered with GI sheets and the building located in Coimbatore. | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Design a purlin on a sloping roof truss with the dead load of 2kN/m2(cladding and insulation), live load of 1.5kN/m2 and a wind load of 3kN/m2(suction). The purlins are 3m center to center and of span 4.5m, simply supported on a rafter at a slope of 20 degrees. | CO4 | 20 |
| 7. | a. | Design the light gauge steel section for the following data. Two channel section without bent lips of size 200x50mm and thickness of the channel is 4mm, Effective span of the simply supported beam is 4m. Determine the load carrying capacity of the beam. | CO6 | 15 |
|  | b. | Describe the advantages and disadvantages of using light gauge steel section. | CO6 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Write the step by step procedure for the analysis of microwave tower. | CO2 | 15 |
|  | b. | Write short notes on plastic hinges? | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | A three span continuous beam ABCD is loaded with ultimate loads as shown in Fig. Determine the required plastic moment of resistance when the beam is of uniform section. 10kN/m 15kN/m 20kN/m 3m 4m 5m A B C D | CO3 | 20 |