Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3008 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SEISMIC ANALYSIS AND DESIGN OF STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

IS Codes permitted for the exam : IS:1893:2016, IS:4326, IS13920:2016

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Differentiate magnitude and intensity. | CO1 | 4 |
|  | b. | Discuss the phenomenon wherein there is gradual accumulation and release of stress and strain. | CO1 | 6 |
|  | c. | “The epicenters of earthquakes tend to be concentrated in narrow zones”. Explain the phenomenon behind this statement. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Differentiate seismometer and seismograph. | CO1 | 4 |
|  | b. | Classify the internal surface of the earth based on P and S waves velocity variation. | CO1 | 6 |
|  | c. | Discuss on the plate margins and earthquake occurrences. | CO1 | 10 |
| 3. |  | Determine the natural frequency and mode shape of a three storey shear building with the following data m1=m2= 1000kg, m3 =800kg, k1=k2= 2000kN/m, k3 =1500 kN/m | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A three storey building located in Delhi has floor height of 3m and weight of m1=m2=3000kN, m3=2000kN, resting on hard rock, designed for school building. Calculate the forces in each level for the data given below. Mode 1 T=0.86; Mode 2 T=0.265; Mode 3 T=0.145. Roof; 1.000; 1.000; 1.000. 2nd floor; 0.904; 0.216; -0.831. 1st floor; 0.716; -0.701; -0.574. | CO2 | 20 |
| 5. | a. | Discuss the failure modes of masonry infill. | CO3 | 10 |
|  | b. | Explain in detail the necessity for having simple configurations in masonry buildings. | CO4 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the design implication to be adopted for seismic resistant design of masonry structures. | CO4 | 10 |
|  | b. | Discuss with a case study the failure of masonry buildings during earthquake. | CO3 | 10 |
| 7. | a. | Explain the lateral load transfer in an RCC structures. | CO2 | 5 |
|  | b. | As per codal provisions discuss the methods of improving the ductility of RCC structure. | CO4 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | “The stiffness of the rigid frame and a ductile frame varies”. Justify the statement with suitable equations. | CO2 | 5 |
|  | b. | Discuss the provisions in the code for design of shear wall and the precautions to be taken in the design. | CO4 | 15 |
|  |  | Compulsory: |  |  |
| 9. | a. | Rcc structures designed as per IS456, needs improvement in seismic resistance. Suggest two appropriate systems with case studies. | CO5 | 10 |
|  | b. | Suggest the situation when building has to be retrofitted. Generate a scheme for retrofitting of an existing Rcc framed structure. | CO6 | 10 |