Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3009 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STABILITY OF STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | State the Stability criteria. | CO1 | 4 |
|  | b. | Develop the Higher order differential equation for column buckling and hence obtain the critical load of a column with fixed ends. | CO4 | 8 |
|  | c. | Determine the critical load of a prismatic cantilever column by Galerkin method or weighted residual. | CO4 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Develop an equation for maximum deflection of an eccentrically loaded column. | CO2 | 10 |
|  | b. | An equilateral triangular section of side “b” and of a material having bilinear stress strain characteristics (E up to yield and ET = (5/6)E beyond yield) forms the cross section of a hinged-hinged column supporting an axial load “P”. Determine the critical load based on i.Tangent modulus theory. ii. Double modulus theory. | CO3 | 10 |
| 3. | a. | Obtain expression for the maximum deflection and maximum moment of a beam –column whose ends are simply supported and is loaded with uniformly distributed load of intensity ‘q’ per unit length and an axial compressive load of ‘Q’ at the ends. | CO2 | 10 |
|  | b. | Determine the symmetric buckling load of the frame shown below using critical equilibrium. Take same EI for beam and columns | CO4 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Obtain an expression for the deflection of a beam – column whose ends are simply supported and is loaded with a concentrated load “Q” at one third span from the left end. Hence obtain the expressions for maximum deflection and maximum bending moment if the concentrated load is applied at mid-span and compare the results with that of a simply supported beam of same span and flexural rigidity and carrying a central concentrated load of same magnitude. | CO2 | 10 |
|  | b. | Determine the buckling load of the frame shown for sidesway buckling using matrix method. Take same EI for beam and columns | CO4 | 10 |
| 5. | a. | Derive the expression for critical stress due to lateral buckling of an I beam in pure bending. | CO2 | 10 |
|  | b. | State the equilibrium equations for buckling of thin walled open section and hence find the buckling loads of a channel section used as a column with fixed ends | CO4 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | A cantilever beam of uniform I cross-section and length ‘L’ is subjected to a lateral load ‘P’ at the free end through the centroid of the section. Find its buckling load. | CO4 | 10 |
|  | b. | Determine the length of the column at which the torsional buckling load will be equal to the flexural buckling load of symmetrical I – section made with plates – Flanges 100 mm x 2 mm and web 200 mm x 2 mm, so that the overall depth is 204 mm. Take E = 200 GPa and G = 80 GPa. | CO2 | 10 |
| 7. | a. | Develop the Plate Buckling equation. | CO2 | 10 |
|  | b. | Develop energy expressions for buckling of plate and hence obtain the critical load of a square plate biaxially compressed with equal uniform compression “N0“per unit length. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Obtain the critical load of a rectangular plate of size ( a x b) with all edges hinged and subjected to uniform axial compressive force of intensity Nx per unit length along the shorter edges. Use Galerkin method of weighted residual. | CO4 | 10 |
|  | b. | Obtain an expression for critical load of a clamped square plate of side “a” subjected to equal uniform biaxial compression “N0“ per unit length . Use Finite Difference method. | CO4 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Distinguish between buckling behavior of column, beam, plate and shell. | CO2 | 5 |
|  | b. | Distinguish between symmetric and asymmetric bifurcation. | CO2 | 5 |
|  | c. | Determine the critical stress for cylindrical shell subjected to uniform external pressure. | CO5 | 10 |