Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3024 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANALYSIS AND DESIGN OF PLATES AND SHELLS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | State the assumption of thin plate. | CO1 | 5 |
|  | b. | Compute the expression for deflection and moment of a simply supported rectangular plate a x b subjected to a sinusoidal loading given by qqosinπxasinπyb. Find the value of maximum deflection. | CO3 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | Examine where Navier equation can be used and why. | CO1 | 5 |
|  | b. | Predict the maximum deflection at the centre of plate if a=b by using Navier’s solution for a simply supported rectangular plate under the influence of concentrated load of magnitude P at the centre of the plate. | CO4 | 15 |
| 3. | a. | Explain the structural action of folded plate roof. | CO1 | 5 |
|  | b. | Construct the design steps using ASCE task committee method for an industrial building of 20m x 30mto be roof with RCC folded plate roofs by suggesting suitable layout of the folded plate roofing system. | CO2 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Sketch the various shapes of the folded plates used in practice. | CO1 | 5 |
|  | b. | Design a folded plate to cover a plan area of 6m x 10m and the effect of relative joint displacement can be ignored. | CO6 | 15 |
| 5. | a. | Interpret the structural behavior of thin shells and folded plates. | CO2 | 8 |
|  | b. | Categorize the various classification of shells with a neat sketch. | CO2 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the membrane theory of shells. | CO3 | 5 |
|  | b. | Design the dome and develop the reinforcement details for a circular water tank of 12m diameter to be roofed with a spherical dome, live load is 0.4 kN/m2. | CO5 | 15 |
| 7. | a. | Illustrate the buckling of cylindrical shell. | CO2 | 5 |
|  | b. | Design a reinforced concrete cylindrical shell to cover a hall of 25 m wide and 120m long. The superimposed load due to water proofing material cover and occasional liveload taken as 1.2 kN/m2 of the surface of the shell. Adopt M20 concrete and Fe415 steel. Design one of the intermediate shells and sketch the details of the reinforcement. | CO5 | 15 |
| (OR) |  |  |  |  |
| 8. |  | Design the hyper shell roof of inverted umbrella type to suit the following data: area to be covered in plan 25m x 25m. Assume liveload of 0.75 kN/m2, use M20 grade concrete and Fe415 steel. | CO5 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | State the advantages of North light shells. | CO2 | 5 |
|  | b. | Illustrate the design procedure of North light shell with an example. | CO4 | 15 |