Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 17CE3038 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : THEORETICAL SOIL MECHANICS |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a | Evaluate the principle effective stress and invariant stresses for the given stress components σ’x= 2,σ’y= 0, σ’z= 4, τxy=1, τxz=3, τyz= 1 all stresses are in tons per square foot | CO3 | 12 |
|  | b | Discover if the following components will permit to be continuous after deformation ԑx= axy2, ԑy=ax2y,ԑz=axy, гxy=0,гyz=az2,гzx=ax2+by2 | CO3 | 8 |
| (OR) |  |  |  |  |
| 2. | a | Formulate the equilibrium equations for a soil body subjected to deformation. | CO2 | 12 |
|  | b | Invent the strain values for the both the plane stress and plane strain conditions for the given value σx= 50MPa, σy=30MPa , σz= 25MPa, τxz=10MPa, E=2.5MPa, µ=0.3 | CO2 | 8 |
| 3. | a | Reframe deviator stress for the triaxial test and oedometer test based on the determination of elastic constants | CO1 | 5 |
|  | b | Prove that i) E= G (3λ+2G) λ +G ii) G= 3KE 9K-E | CO2 | 5 |
|  | c | Recall the different Correlative test. | CO1 | 5 |
|  | d | Infer the generalized Hooke’s law and its different form. | CO3 | 5 |
| (OR) |  |  |  |  |
| 4. | a | Recognize the various laboratory methods for the determination of elastic constants. | CO1 | 12 |
|  | b | Assess the intensity of vertical pressure and shear stress at a point 5m directly below a 50kN point load acting at a horizontal surface. What will be vertical pressure and shear stress at a point 3m away from the axis of loading but at the same depth of 5m? | CO4 | 8 |
| 5. | a. | Compare Flamant’s problem and Kelvin’s problem. | CO5 | 7 |
|  | b. | Solve Flamant’s problem for line load of 10kN/m at a point 10m below ground level, with radial distance 4m assume suitable values for soil properties. | CO5 | 5 |
|  | c. | Estimate the stresses and displacement at a point 6m below the ground level for a point load of 100kN acting at a radial distance of 3m assume υ = 0.3 and E = 3MPa. | CO3 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | A square footing 10ft, on each side, resting on the surface of a normally consolidated 40ft thick clay layer underlain by bedrock. The properties of the clay before loading are c= 250psf, ϕ= 0, Cc=0.4, γ = 100pcf initial void ratio e = 0.73. The water table is at the surface of soil .Estimate the maximum total settlement of the footing under a uniformly distributed load of magnitude such that the factor of safety of the footing with respect to bearing capacity is 3.0. | CO4 | 12 |
|  | b. | Predict the relationship between ko, the coefficient pressure at rest and the elastic constants. | CO4 | 8 |
| 7. | a. | Evaluate the stability of soil in vertical bank using the limiting equilibrium and limit analysis method. | CO5 | 10 |
|  | b. | Explain briefly the upper bound theorem and lower bound theorem in limiting equilibrium. | CO5 | 5 |
|  | c. | Illustrate any insitu test for determining coefficient of permeability. | CO5 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss different methods in which flownets can be plotted. | CO6 | 15 |
|  | b. | Measure the average coefficient of permeability in horizontal and vertical direction for a deposit of three layers of thickness 4m, 2m, and 3m and having the coefficient of permeability of 3x10-2mm/sec, 2x10-5 mm/sec and 3x10-3mm/sec. Assume the material is isotropic. | CO6 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe the seepage flow through isotropic and anisotropic material. | CO6 | 8 |
|  | b. | Demonstrate Permeability test using rowe type consolidation cell and flexible wall permeameter. | CO6 | 6 |
|  | c. | Explain the graphical method for the determination of flow net. | CO6 | 6 |