Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : 17CE3039 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : DEEP FOUNDATIONS |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Enumerate the general considerations and functions of pile foundation. | CO1 | 10 |
|  | b. | Define deep foundation classifications and its types. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Consider a pile having an outside diameter of 406 mm. the embedded length of the pile in layered saturated clay is 30m the following are the details of the subsoil: Depth (m); Unit weight ᴦ(kN/m3); Cu (kN/m2). 0-5; 18; 30. 5-10; 18; 30. 10-30; 19.6; 100. The water table is located at a depth of 5m from the ground surface. Estimate Qp by using i. Mayerhof’s method. ii. Vesic’s method. | CO1 | 10 |
|  | b. | List the advantages and disadvantages of cast in-situ piles over driven piles. | CO3 | 10 |
| 3. | a | Define pile cap with neat sketch. | CO1 | 5 |
|  | b | Summarize Types and structural design of pile. | CO2 | 15 |
| (OR) |  |  |  |  |
| 4. | a | Elucidate the term drilled caissons. | CO1 | 5 |
|  | b | Enumerate the wave equation analysis. | CO2 | 15 |
| 5. | a | List out the problems on pile foundation in sand ,clay , silt loose and non uniform soils. | CO4 | 10 |
|  | b | Differentiate skin friction and sinking effect. | CO5 | 10 |
| (OR) |  |  |  |  |
| 6. | a | Explain the elastic settlement on pile group. | CO4 | 10 |
|  | b | Consider a 3 x 4 group of pre stressed concrete piles, each 21 m long, in a sand layer. the working load for pile group is 6024 kN(3 x4 x Qall; where Qall =502kN ) and d/D =3. Estimate the elastic settlement of the pile group. | CO5 | 10 |
| 7. | a. | Consider a concrete pile that is 0.305m x 0.305m in cross section in sand. The pile is 15.2m long. The following are the variations of N60 with depth. Design the reinforcement and detailing. Depth below ground level (m); N60. 1.5; 8. 3.0; 10. 4.5; 9. 6.0; 12. 7.5; 14. 9.0; 18. 10.5; 11. 12.0; 17. 13.5; 20. 15.0; 28. 16.5; 29. 18.0; 32. 19.5; 30. 21.5; 27. | CO2 | 15 |
|  | b. | Detail about pile driving equipments. | CO5 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Discus about pile driving and pile installation mechanism. | CO5 | 5 |
|  | b. | Differentiate the types of piles with neat sketch. | CO3 | 15 |
|  |  | Compulsory: |  |  |
| 9. | a. | Define well foundation and its elements. | CO6 | 5 |
|  | b. | List the types of coffer dams and and deep soil deposites. | CO6 | 5 |
|  | c. | Elucidate lateral pressure stability & pinning. | CO5 | 5 |
|  | d. | Infer pier underpinning. | CO5 | 5 |