Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3040 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FINITE ELEMENT METHODS FOR GEOTECHNICAL ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define the Principle of virtual work. | CO1 | 2 |
|  | b. | Develop the St. Venant compatibility condition for a plane stress problem. | CO2 | 4 |
|  | c. | Develop the Generalized Hook’s law for an isotropic material. | CO2 | 4 |
|  | d. | The following differential equation available for a physical phenomenon. d2ydx2+500x2=0 0 ≤ x ≤ 1 By using the trial function y = a (x – x3) + b (x – x5), calculate the value of the parameters “a” and “b” by the Galerkin method of weighted residual. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Distinguish between Strong formulation and Weak formulation. | CO2 | 2 |
|  | b. | Explain the Principle of Stationery potential energy. | CO2 | 4 |
|  | b. | Explain the Rayleigh Ritz method. | CO2 | 6 |
|  | d. | Develop the Differential equations of equilibrium in 3D rectangular coordinates. | CO2 | 8 |
| 3. | a. | Distinguish between discrete finite element and continuum finite element with examples. | CO2 | 2 |
|  | b. | Develop the shape functions for a 6 noded planar triangular element and prove that it is LST. | CO3 | 8 |
|  | c. | Develop the Shape functions for a 2D Euler beam element and hence the stiffness matrix. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Distinguish between global coordinate, local coordinate and natural coordinate. | CO2 | 2 |
|  | b. | Explain the basic philosophy of finite element and the criteria to be satisfied by a displacement model for monotonic convergence. | CO3 | 8 |
|  | c. | Using the Principle of virtual work, develop the force displacement matrix for a two noded axial element oriented at an angle “ϴ” measured from positive x axis in the anticlockwise direction. Area of cross section and length of the element are A and L respectively. Take E as Young’s Modulus of the material of the element. | CO3 | 10 |
| 5. | a. | List the theorems in isoparametric element. | CO1 | 2 |
|  | b. | Distinguish between Serendipity and Lagrangean family of elements. | CO2 | 2 |
|  | c. | Develop the shape functions for 8 noded serendipity element | CO3 | 6 |
|  | d. | Develop the stiffness matrix and consistent load vector for an iso parametric quadrilateral element. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain Static Condensation Technique. | CO2 | 4 |
|  | b. | Evaluate I=13dxx using Gaussian three point formula. | CO3 | 6 |
|  | c. | Using two point Gaussian quadrature function, evaluate the integral I=y=c=4y=d=6x=a= −2x=b=2(1−x)2(4−y)2dxdy | CO3 | 10 |
| 7. | a. | Develop the constitutive law for an axisymmetric stress analysis. | CO2 | 4 |
|  | b. | Develop the shape functions and jacobian for a 8 noded hexahedral element. | CO3 | 6 |
|  | c. | Develop the stiffness matrix and consistent load vector for a 4 noded tetrahedran element. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain (i) materially nonlinear problem (ii) Geometrically nonlinear problem and Both materially and geometrically nonlinear problem. | CO6 | 4 |
|  | b. | Describe the development of rectangular plate bending element. | CO6 | 6 |
|  | c. | Explain the various classess of shell elements and their limitations. | CO6 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Develop the finite element model for analysis of Terzaghi’s one dimensional consolidation problem. | CO6 | 5 |
|  | b. | Discuss the requirements of a commercial software for stability analysis of geotechnical structures such as earth dam, embankment and cut slopes. | CO6 | 5 |
|  | c. | Discuss the step by step by step finite element analysis procedure to evaluate the steady state seepage flow under a dam structure. The head of water on the upstream side is 10 meter and zero on the downstream side of the dam. Use 4 noded isoparametric quadrilateral element. The impervious zones are shown hatched. | CO6 | 10 |