Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 17CE3041 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : SOIL DYNAMICS AND MACHINE FOUNDATIONS |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a | Show that how uniform circular motion executes simple harmonic motion. | CO1 | 7 |
|  | b | List out and explain different types of vibration. | CO1 | 8 |
|  | c | A vertical cable 2 m long has a cross sectional area of 5 cm2 supports a weight of 100 kN. What will be the natural period and natural frequency. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Recall the Principles of vibration measuring instruments. | CO1 | 12 |
|  | b. | Elaborate the conditions for different damping condition with equations. | CO1 | 8 |
| 3. | a. | Recognize the relevance of vibration table studies and its major conclusion. | CO2 | 5 |
|  | b. | Enumerate the anti liquefaction measures. | CO2 | 10 |
|  | c. | Describe how to evaluate the liquefaction potential using SPT. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. |  | Explain the various field test for determining the dynamic properties of soil. | CO2 | 20 |
| 5. | a. | Infer the various machines with its foundation arrangements. | CO3 | 8 |
|  | b. | Identify the general design procedures for the foundation of reciprocating machine. | CO4 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | A reciprocating machine is symmetrically mounted on a block of size 5.0 m x 4 m x 4 m high. The soil at the site is sandy in nature having ϕ = 30° and γsat= 10 kN/m2. The water table liesat adepth of 3.0 m below the ground surface. The block is embedded in the ground by 2.0m depth. The machine vibrating at aspeed of 250rpm generates Maximum vertical unbalanced force=2.5kN Torque about Z axis = 4 kNm Maximum horizontal unbalanced force = 2kN at a height of 0.2m above the top of the block. Assume the limiting amplitude of machine as 150microns and Cu = 3.5x104kN/m2 , G= 1x104kN/m2,E= 3x104kN/m2 and µ = 0.35 Determine the natural frequencies and amplitude using weightless spring method | CO4 | 15 |
|  | b. | Label the different methods for analysis of machine foundation. | CO4 | 5 |
| 7. | a. | Memorize the special consideration for the design of turbo generator foundation. | CO5 | 10 |
|  | b. | Illustrate the various arrangements of foundation for impact machine with suitable diagrams. | CO5 | 10 |
| (OR) |  |  |  |  |
| 8. |  | A 10 kN forging hammer is proposed to install in a factory. The hammer has the following specification Weight of tup without tie= 11 kN, maximum tup stroke= 700 mm, weight of upper half of the die=40 kN, area of piston = 0.12 m2, supply steam pressure= 650 kN/m2, weight of anvil block= 250 kN, total weight of hammer= 350 kN, bearing area of anvil =2mx2m, permissible vibration amplitude for anvil= 2mm, permissible amplitude for vibration= 1.5mm. It is proposed to use pine wood pad of thickness 0.5m below the anvil. The modulus of elasticity of pad material is 5x105 kN/m2, and allowable compressive stress in pad is 3500 kN/m2 A vertical resonance test was done on a 1.5mx0.75mx0.7m high concrete block at the proposed depth of foundation. The data obtained are given below ɵ (deg); fnz(Hz); Amplitude at resonance (microns). 36; 41; 13. 72; 40; 24. 108; 34; 32. 144; 31; 40. The soil at the site is sandy in nature and water table lies at a depth of 3.0m below ground level allowable soil pressure is 225 kN/m2. Design a suitable foundation for the hammer. | CO5 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Identify the different types of vibration isolation and explain the different materials used for isolation. | CO6 | 10 |
|  | b. | List out the important considerations with vibration isolator selection. | CO6 | 5 |
|  | c. | Discuss about the active screening using circular trenches. | CO6 | 5 |