Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 17CE3042 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : GROUND IMPROVEMENT TECHNIQUES |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the scope for ground improvement techniques in the present scenario. | CO1 | 10 |
|  | b. | Explain the process of consolidation by electro osmosis method. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | How does dewatering help in improving the geotechnical properties of soil. | CO1 | 10 |
|  | b. | Explain in detail the well point system of drainage. | CO | 10 |
| 3. |  | What are the methods used in the field compaction of soil. Explain any two methods suitable for cohesionless soil. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A site close to a river is prone to earthquake. The ground water table is close to the ground level. The results of the grain size analysis tests on a soil specimen from 5 m below ground level are shown in the Table. Evaluate the suitability of the soil for densification by vibro-compaction Particle size, mm; 2; 0.84; 0.425; 0.25; 0.106; 0.075; 0.06; 0.025. % finer by weight; 100; 94.1; 83.8; 72.6; 57.8; 44.7; 35.8; 29.7. | CO2 | 20 |
| 5. |  | A 5 m thick highway embankment is to be constructed at a location where the soil profile is 6m thick normally consolidated clay with sand lenses over rock. The undrained strength of the clay is 15 kPa. The unit weight of embankment material is 18 kN/m3 . Stone columns 90 cm in diameter are proposed to be used in equilateral triangle grid pattern at 2.5 m center-to-center spacing. Determine: i) the load carried by a single stone column, and ii) the safety of the stone column in load bearing. | CO5 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain the step by step procedure for the design of stone column and its installation in soil. | CO5 | 20 |
| 7. | a. | Under what situations electrical stabilization is recommended. | CO4 | 5 |
|  | b. | Describe the thermal and freezing techniques in stabilization of soil. | CO4 | 15 |
| (OR) |  |  |  |  |
| 8. |  | Explain stabilization of soil by the following methods - Nano stabilization. - Bio stabilization. | CO4 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Summarize any two types of grouting techniques and compare its merits and demerits. | CO6 | 20 |