Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 17CE3043 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : PAVEMENT ENGINEERING |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Distinguish between a highway pavement and an airport pavement. Can the design method used for highway pavement be used for airport pavement? Why? | CO1 | 10 |
|  | b. | Summarize the functions of base and subbase course in a flexible pavement. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Sketch the structural cross section of flexible pavement and indicate the materials used in each layer with its sizes as per Indian Road Congress. | CO1 | 10 |
|  | b. | Explain about the stresses and deflection of a pavement using two layer theory. | CO3 | 10 |
| 3. |  | Design the thickness of pavement layer of a flexible pavement as per IRC 37 for the given datas. - Two lane carriageway - Plain terrain - CBR = 4% - Initial traffic (both the directions) = 1000CV/day - Design life = 20years - VDF = 2.5 - Rate of growth = 7.5% | CO4 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Enumerate the limitations of CBR method of design of pavements. | CO2 | 10 |
|  | b. | Explain with a sketch the Hveem stabilometer method of obtaining ‘R’ in a pavement material. Also find the two extreme values of R value. | CO3 | 10 |
| 5. |  | A corner of a 200mm thick concrete slab resting on a soil of modulus of subgrade reaction 30MN/m3 is subjected to a wheel load of 41kN with a tyre pressure of 0.6 MPa during night time of s peak winter. Find the nature and magnitude of resultant stress at the top of the slab using Bradbury’s equations. Also determine the nature and magnitude of stress due to subgrade restraint at the bottom. The other relevant data are given below Modulus of elasticity of concrete : 3 x 104 MPa Poisson’s ratio of concrete : 0.15 Coefficient of thermal expansion of concrete : 10 x 10-6/o C Unit weight of concrete : 25kN/m3 Temperature differential : 1oC/10 mm thickness of slab Coefficient of friction between the slab and soil :1.5 Spacing of contraction joints :4.5m | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A 340-mm thick concrete pavement designed for a two-way National Highway in Coimbatore is found to satisfy the requirement of cumulative fatigue life consumed for a particular set of data . Check as pe IRC : 58 – 2002.whether this thickness is adequate for satisfying the following requirements - Maximum axle load stress at edge + warping stress at edge < Flexural strength of concrete - Coner wheel load stress for 98th percentile axle load < Flexural strength of concrete The other relevant data are given below Flexural strength of concrete : 4.5 MPa Modulus of subgrade reaction : 80MN/m3 Modulus of elasticity of concrete : 3 x 104 MPa Poisson’s ratio of concrete : 0.15 Coefficient of thermal expansion of concrete : 10 x 10-6/o C Tyre Pressure : 0.8MPa Spacing of contraction joints :5.0m Unit weight of concrete : 25kN/m3 Maximum factored single axle load : 24t 98th percentile single axle load : 16t | CO4 | 20 |
| 7. | a. | How is serviceability index of ural roads estimated? | CO2 | 10 |
|  | b. | Explain how the thickness of flexible overlay over rigid pavement is calculated. | CO5 | 10 |
| (OR) |  |  |  |  |
| 8. |  | The rebound deflection values (in mm) obtained in Benkleman Deflection method on a flexible pavement of a National Highway are : 1.25, 127, 1.40, 1.30, 1.35, 1.20, 1.25, 1.37, 1.28. Find as per IRC: 81-1997,the required thickness of Dense Bituminous Macadam overlay for the following data. Two lane sinle carriageway Pavement Temperature : 39oC Liquid limit of the clayey subgrade : 45% Plastic limit of the subgrade : 20% Moisture content of subgrade :11% Average annual rainfall : 900mm Number of commercial vehicles per day : 3000 Vehicle damage factor :4.66 Annual traffic growth rate :7.5% Design life :15 years | CO4 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the applications of geofabrics in road constructions based on its functions. | CO6 | 20 |