Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3058 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELEMENTS OF HYDROLOGY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain radiation and the effect of radiation over earth surface and climate. | CO1 | 7 |
|  | b. | A lake has an area of 15km2. Observation of hydrological variables during a certain year are precipitation = 700mm/year, Average inflow Qin = 1.4m3/s, Average outflow Qout=1.6m3/s. Assume that there is no water exchange between the lake and the groundwater. Determine the evaporation during this year. | CO4, CO6 | 13 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the factors influencing the infiltration. | CO4 | 8 |
|  | b. | A 7-hour storm produced the following rainfall intensities (mm/hr) at half an hour intervals over a basin of area 1830 km 2 is 4, 9, 20, 18, 13, 11, 12, 2, 8, 16, 17, 13, 6 and 1. If the corresponding observed runoff is 36.6 million m3 , estimate Φ – index for the storm. | CO4 | 12 |
| 3. | a. | Explain the various sources of stream flow and types of flow occurring on the surface due to this. | CO2 | 10 |
|  | b. | Which is the most suitable routing method for deriving a discharge-storage relationship in a river? Elaborate the Method. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Given below are the observed flows from a storm of 4-h duration on a stream with a drainage area of 1600 km2. Derive and plot the 4-hr unit hydrograph assuming a constant base flow of 100 m3/sec. Time(day); 1; 2; 3; 4; 5; 6; 7; 8; 9. Flow(m3/sec); 100; 1000; 830; 630; 520; 420; 350; 280; 218. Time(day); 10; 11; 12; 13; 14. Flow(m3/sec); 180; 155; 130; 110; 100. | CO2 | 20 |
| 5. | a. | A confined aquifer has 3 layers. The bottom layer consists of coarse sand of thickness 4 m, having hydraulic conductivity of 0.01 cm/sec. The middle layer consists of fine sand of thickness 3 m, having hydraulic conductivity of 0.002 cm/sec. The top layer consists of gravel of 4 m having hydraulic conductivity of 2 cm/sec. Calculate the equivalent hydraulic conductivity of the confined aquifer if: - the flow is along the stratification. - the flow is normal to the stratification. | CO5 | 15 |
|  | b. | Define the following terms: - Cone of depression and radius of influence. - Specific capacity of the well. | CO5 | 5 |
| (OR) |  |  |  |  |
| 6. |  | Using Jacob method, obtain S and T for a given aquifer. The time drawdown data is given in the table for which the observation well is 12.3 m from the pumping well. Time in (min); Drawdown s (m). 1; 0.24. 2; 0.24. 3; 0.28. 4; 0.32. 6; 0.37. 8; 0.41. 10; 0.45. 14; 0.49. 18; 0.51. 22; 0.53. 28; 0.54. 35; 0.57. 45; 0.64. 55; 0.65. 65; 0.68. 80; 0.69. 100; 0.74. 120; 0.76. | CO5 | 20 |
| 7. | a. | A fully penetrating artesian well of 0.3 m diameter was pumped with a constant rate of 1800 m3/day under a steady state condition. If the transmissivity of the aquifer is 0.145 m2/min and the drawdown observed in the well is 8.92 m, find the radius of influence of the well. | CO5 | 10 |
|  | b. | In an area of 100ha, the water table dropped by 4.5m. If the porosity is 30% and the specific retention is 10% determine: - the specific yield of the aquifer - change in ground water storage. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Differentiate between the following terms: - Confined and unconfined aquifers. - Aquifuge and aquiclude. | CO3 | 6 |
|  | b. | Explain the role of groundwater in hydrological cycle. | CO3 | 7 |
|  | c. | Write short notes on groundwater budget. | CO5 | 7 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the water budget method of reservoir evaporation estimation. | CO1 | 6 |
|  | b. | Estimate the daily evaporation from a water surface using the Penman’s approach, at a place of Latitude 20º N from the following Data. Air temperature = 26º C; Saturation vapour pressure at 26º C = 33.608 mb; Vapour pressure of air = 13.25 mm of mercury; Number of sunshine hours = 7.6; Maximum probable bright sunshine hours =13.2; Solar radiation = 958 cal/cm/day; Wind velocity at 2 m height = 2.5 m/s; Atmospheric pressure = 752 mm of mercury. | CO1 | 14 |