Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17CE3059 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF HYDRAULIC AND CONVEYANCE STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | List different types of canals. | CO3 | 5 |
|  | b. | Define a canal outlet. Discuss any one type. | CO2 | 5 |
|  | c. | State the limiting height of a gravity dam. | CO1 | 5 |
|  | d. | List the types of spillways? | CO2 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss design principle of a channel using Lacey’s method. | CO3 | 10 |
|  | b. | Design an irrigation channel based on Kennedy’s theory with the following details Discharge : 60 cumec Bed Slope : 1 in 6000 Critical Velocity Ratio m : 1.05 Rugosity coefficient : 0.02 | CO3 | 10 |
| 3. | a. | How the cross drainage works are classified? Discuss with neat diagrams. | CO6 | 10 |
|  | b. | A masonry dam 10 m high is trapezoidal in section with a top width of 1m and bottom width of 8.25m. The upstream face has a batter of 1:10. The water is stored up to 10m height. There is no downstream water. Find the factor of safety against overturning. Consider water force, self weight and full uplift pressure. Assume unit weight of masonry and water suitably. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Discuss the classification of earth dams in detail with neat sketches. | CO2 | 10 |
|  | b. | List out different types of spillway gates and discuss them with neat diagrams. | CO5 | 10 |
| 5. | a. | Discuss ogee spillway. | CO4 | 5 |
|  | b. | List the types of diversion head works. | CO5 | 5 |
|  | c. | Discuss the design principles of channels by Kennydy’s methods. | CO6 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain different causes of failure of earth dam. | CO1 | 5 |
|  | b. | Differentiate between head regulator and cross regulator. | CO4 | 5 |
|  | c. | Enlist the factors to be decide, while selecting a site for a resvoir. | CO4 | 5 |
|  | d. | Paraphrase the suitable conditions for dam site. | CO5 | 5 |
| 7. | a. | Explain canal lining and state the economic benefits. | CO6 | 5 |
|  | b. | Define fall. When is it necessary to construct a fall? | CO2 | 5 |
|  | c. | Explain the procedure to find out hydraulic gradient below the weir on permeable foundation. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Classify various types of the dams. | CO1 | 10 |
|  | b. | List various failures of gravity dam. | CO3 | 5 |
|  | c. | What are the uses of canal lining and the importance. | CO6 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Differentiate between weir and barrage. Explain with neat sketch | CO3 | 5 |
|  | b. | Explain how energy dissipation arrangements are made below a spillway. | CO5 | 10 |
|  | c. | What are the measures to control seepage in earth dams? | CO2 | 5 |