Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 17CE3060 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : WATER RESOURCES PLANNING AND SYSTEMS # ENGINEERING |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions |  | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. | Define system. Enumeratethe different types of system models used in analysis of water resources systems. |  | CO5 | 10 |
|  | b. | “The basic factor in design and operation of a multipurpose project is compromise” – substantiate the statement (with an example) |  | CO1 | 5 |
|  | c. | What are the characteristics of water resources planning/management problems? Describe which are the most suitable for quantitative systems analysis techniques. |  | CO1 | 5 |
| (OR) |  |  |  |  |  |
| 2. | a. | 1. Explain the characteristics of linear programming 2. Select project from the set of alternative plans, as stated below: i. the least initial investment is to be selected ii. the least operating and maintenance cost is to be selected iii. the lowest annual cost is to be preferred. iv. the lowest interest rate is to be preferred |  | CO4 CO3 | 3 3 |
|  | b | By using the graphical method, solve the following linear programming, Maximize z = 5x1 + 4 x2 Subject to 6x1 + 4 x2<= 24 x1+2x2<=6 -x1+x2<=1, x2<=2, x1,x2>=0 |  | CO5 | 8 |
|  | c | Describe the mass curve method of determining the storage capacity of a reservoir to be designed for a uniformly steady demand rate |  | CO6 | 6 |
| 3. | a. | With the help of neat sketches, explain briefly the following - Local maximum - Global maximum - Global minimum - Strictly concave function - Saddle point |  | CO4 | 10 |
|  | b. | Write a detailed note on the cost allocation for multipurpose projects in economic analysis of projects. |  | CO3 | 5 |
|  | c | Discuss with the help of appropriate examples, the environmental considerations necessary in water resources planning. |  | CO1 | 5 |
|  |  |  | (OR) |  |  |
| 4. |  | Solve the following problem by using traditional Big- M simplex tableau method Maximize Z = -3 X1+X2+X3 subject to 1X1-2X2+X3<=11 -4X1+1X2+2X3>=3 2X1+0X2-1X3 = -1, X1>=0, X2>=0, X3>=0 |  | CO5 | 20 |
| 5. | a. | i. A linear system is described by ‘m’ equations and ‘n’ variables. Identify the system for the following cases: - m>n (b) m<n ii. Write the dual formulation of the following Maximize Z = cx subject to Ax<=b, x>=0 |  | CO4 | 5 |
|  | b. | In a town, it was decided to expand its water supply system with an existing capacity of 10 units to the ultimate requirement of 40 units by the end of 15 years from now, in stages of 5 years each. The present cost of expansion at each stage is estimated to be equal to the square of the number of units added at that stage. The capacity requirement is estimated to be 15, 25 and 40 units by the end of 5, 10 and 15 years from now. Determine how many units should be added at each stage for minimum total cost of capacity expansion over 15 year planning horizon. Capacity can be added only in 5-unit increments. |  | CO6 | 12 |
|  | c. | Define Bellman’s principle of optimality. State the main drawback of the dynamic programming. |  | CO5 | 3 |
| (OR) |  |  |  |  |  |
| 6. | a. | Area of a farmer’s field is 200 hectares (ha) and he wants to grow wheat and pulses on his land. It costs Rs. 3/ha to raise wheat crop while for pulses, the cost is Rs.1/ha. The benefit from wheat crop is Rs.5/ha and pulses crop, is Rs 2/ha. A sum of Rs. 300 is available for raising both crops. Formulate a linear programming to find the optimal cropping plan (how much area to be allotted to wheat crop and pulses) so as to maximize the total net benefits. |  | CO4 | 11 |
|  | b | Discuss about the planning arrangements for irrigation water projects and related data requirement. |  | CO1 | 5 |
|  | c | List the important tasks for planning water resources project. |  | CO1 | 4 |
| 7. | a. | Discuss in detail about the constitutional provision guidelines in water resources planning and development in our country. |  | CO3 | 8 |
|  | b | Explain the term ‘planning’ in the light of water resources planning. |  | CO1 | 4 |
|  | c | Discuss in detail about the optimization techniques applied in reservoir operation and water allocation problems (write the objective function, stages and state of the system) |  | CO4, CO5, CO6 | 8 |
| (OR) |  |  |  |  |  |
| 8. | a. | Enumerate the list and how the funding agencies are responsible for implementing and monitoring the water resources projects List down the central agencies in water resources sector |  | CO 3 | 8 |
|  | b. | State important features of various classes of reservoirs and write short notes on reservoir losses |  | CO1 | 5 |
|  | c | State the objectives of developing water resources projects of national interest? |  | CO2 | 4 |
|  | d | Explain the basic difference between ‘optimization’ and ‘Simulation’ in water resources planning? Is optimization possible through simulation? |  | CO5 | 3 |
|  |  | Compulsory: |  |  |  |
| 9. | a. | Discuss in detail about various components of National Water Policy and enumerate their significance and implementation in the context of water resources planning and development. |  | CO2 | 16 |
|  | b. | List down the concerns of National Water Policy in the present scenario. |  | CO2 | 4 |