Reg.No. ____________

End Semester Examination – April/May 2018

| # Code : | # 17EC3005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WIRELESS COMMUNICATION NETWORKS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Find the outage probability at 15 m for a channel based on combined path loss and shadowing models [KdB=-31dB & ɤ=3.71]. Assume the transmit power of Pt =10mW and minimum power requirement of Pmin=-110.5dBm. | CO1 | 8 |
|  | b. | Consider a receiver with noise power -160 dBm within the signal bandwidth of interest. Assume a simplified path loss model with d0 = 1 m, K obtained from the free space path loss formula with Omni-directional antennas and fc = 1 GHz, and γ = 4. For a transmit power of Pt = 10 mW, find the maximum distance between the transmitter and receiver such that the received signal-to-noise power ratio is 20 dB. | CO1 | 8 |
|  | c. | Build the log-normal shadowing distribution for fading signals in the wireless channels. Explain why Gaussian distribution is used to model attenuation due to blocking objects? | CO2 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Formulate the Link Power budget analysis of Large scale propagation model? Detail the significance factors involved in it. | CO1 | 10 |
|  | b. | Consider a cellular system operating at 900 MHz where propagation follows free space path loss with variations from log normal shadowing with σ = 6 dB. Suppose that for acceptable voice quality a signal-to-noise power ratio of 15 dB is required at the mobile. Assume the base station transmits at 1 W and its antenna has a 3 dB gain. There is no antenna gain at the mobile and the receiver noise in the bandwidth of interest is -10dBm. Find the maximum cell size so that a mobile on the cell boundary will have acceptable voice quality 90% of the time. | CO2 | 10 |
| 3. |  | Consider a flat-fading channel of bandwidth 20 MHz where for a fixed transmit power S, the received SNR is one of six values: γ1 = 20 dB, γ2 = 15 dB, γ3 = 10 dB, γ4 = 5 dB, and γ5 = 0 dB and γ6 = −5 dB. The probability associated with each state is p1 = p6 = .1, p2 = p4 = .15, p3 = p5 = .25. Assume only the receiver has CSI. Find the Shannon capacity of this channel. Plot the capacity versus outage for 0 ≤ pout < 1 and find the maximum average rate that can be correctly received (maximum Co). | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Determine the average probability of bit error in Maximal ratio combining and Equal gain combining techniques. | CO3 | 10 |
|  | b. | Find the outage probability of BPSK modulation at Pb = 10−3 for a Rayleigh fading channel with SC diversity for M = 1 (no diversity), M = 2, and M = 3. Assume equal branch SNRs of γ = 15 dB. | CO4 | 10 |
| 5. |  | For the 4 × 4 MIMO channels given below, find their capacity per unit Hz assuming both transmitter and receiver know the channel, for channel SNR ρ = 10 dB. H1=11−1111−1−11111111−1 H2=111−111−111−1111−1−1−1 | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Consider a channel with channel matrix. H=.1.5.9.3.2.6.1..3.7 Assuming ρ = 10 dB, find the output SNR when beamforming is used on the channel with equal weights on each transmit antenna and optimal weighting at the receiver. Compare with the SNR under Beamforming with optimal weights at both the transmitter and receiver | CO4 | 15 |
|  | b. | Find the SVD of the following matrix.3 H=.7.6.2.4.1.5.9.2.3.6.9.1 | CO4 | 5 |
| 7. |  | Illustrate the random acess scheme for multi user systems in detail. | CO5 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Compare TDMA, CDMA and FDMA. | CO5 | 10 |
|  | b. | Describe the uplink and downlink system model for multi user communications. | CO5 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe on IEEE 802.11a/g –Orthogonal Frequency Division Multiplexing-Based Local Area Networks in details. | CO6 | 10 |
|  | b. | Elaborate on packet transmission in Wireless Local area network. | CO6 | 10 |