Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code :17EC3018 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : ADVANCED DIGITAL IMAGE PROCESSING |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Summarize the fundamental steps involved in digital image processing. | CO1 | 15 |
|  | b. | Illustrate the concept of perception. | CO6 | 5 |
| (OR) |  |  |  |  |
| 2. |  | Illustrate the application of Digital image processing with a real time example. | CO4 | 20 |
| 3. | a. | Justify the need for image transforms and write the DFT forward and reverse equation with its property. | CO1 | 15 |
|  | b. | Differentiate spatial and intensity resolution. | CO6 | 5 |
| (OR) |  |  |  |  |
| 4. |  | Demonstrate the spatial filtering process with an example and explain the types and advantages of various spatial smoothening filter. | CO5 | 20 |
| 5. | a. | Analyze the smoothening image enhancement filtering techniques in frequency domain. | CO5 | 15 |
|  | b. | Estimate the transfer function of 2nd order Butterworth high pass filter for Do Value of 20 and D (u,v) values of 10, 20 and 30. Plot the graph between H(u,v) and D(u,v). | CO5 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the edge detection and modelling with suitable examples. | CO3 | 15 |
|  | b. | Predict the output image pixel values for the given sample using Erosion morphological process. Input image : 1; 0; 0; 1; 1; 1; 1; 0; 0. Structuring element: 1; 1; 1. | CO2 | 5 |
| 7. | a. | Outline the transformation function used in Image registration process. | CO5 | 15 |
|  | b. | Outline on thresholding Algorithm. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. |  | Analyze the pixel and region based fusion techniques adopted in processing the digital images. | CO6 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Summarize the concepts and performance of 3D image processing with suitable examples. | CO4 | 20 |