Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17EC3039 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # VLSI DIGITAL SIGNAL PROCESSING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Mention the classification of the devices depending upon granularity. Explain all the types with examples. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | List few advantages of FPL. | CO1 | 6 |
|  | b. | Show the classification of VLSI circuits. | CO1 | 6 |
|  | c. | State the applications of Digital signal processing. | CO1 | 8 |
| 3 | a. | State Data broadcast structure for 3 tap FIR filter to reduce the critical path without introducing any pipelining latches. | CO4 | 5 |
|  | b. | Find values of S’UV ,W(U,V) and D(U,V) for the following DFG. | CO2 | 15 |
| (OR) |  |  |  |  |
| 4 | a. | Find the equations for y(3k),y(3k+1),y(3k+2) following parallel processing 3 tap FIR filter. | CO2 | 5 |
|  | b. | Unfold the following switch. unfolding factor J=3 | CO2 | 15 |
| 5 | a. | Derive a 2x2 convolution algorithm using modified Cook Toom algorithm with β=0,+ 1 where h(p)=h0+h1p, x(p)=x0+x1p and s(p)=s0+s1p+s2p2 | CO3 | 15 |
|  | b. | Interpret the Clustered Look Ahead pipelining in IIR filter. | CO4 | 5 |
| (OR) |  |  |  |  |
| 6 | a. | Mention three forms of pipeline interleaving in Digital Filters and discuss any one form in detail. | CO4 | 12 |
|  | b. | Draw a 4 parallel 1st order recursive filter. | CO2 | 8 |
| 7 | a. | Consider the following 3rd order polynomial D3Z=345.1Z3−819.3Z2+665.8Z−183.6 Using Schur algorithm calculate Schur polynomial. | CO6 | 10 |
|  | b. | Explain the working of any two parallel multipliers with its structure. | CO5 | 10 |
| (OR) |  |  |  |  |
| 8 | a. | Find output roundoff noise for the l2 scaled filter for the following 2nd order filter SFG. A=011160 b=01 c=116−12 d=0 | CO6 | 15 |
|  | b. | Mention the digit sets involved in Hybrid Radix-2 addition. Also draw eight bit hybrid radix-2 adder. | CO5 | 5 |
| Compulsory: |  |  |  |  |
| 9 | a. | Consider the 4 tap FIR filter yn=1.01010000010∗xn+0.10001010101∗xn−1+0.10010000010∗xn−2+1.00000101000∗xn−4 Using substructure matching procedure, obtain the reduction in the number of operations. | CO6 | 15 |
|  | b. | Explain any two power reduction techniques in digital circuit design. | CO5 | 5 |