Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17EE1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASIC ELECTRICAL ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A alternating sinusoidal current equation is given by i = 200sin314t. Obtain Iavg, Irms, form factor, peak factor, frequency and time period. | CO1 | 10 |
|  | b. | Using Kirchhoff’s laws, find the current in various resistors in the circuit shown below. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | State Ohm's law and mention its limitations. | CO1 | 5 |
|  | b. | Obtain an equivalent voltage source for the following circuit. | CO1 | 5 |
|  | c. | An electrical network is arranged as shown in the following figure. Find the a) total equivalent circuit resistance (6 Marks); b) total current in branch AF. (4 Marks) | CO1 | 10 |
| 3. | a. | Find the magnetic field intensity (H) and self inductance (L) of a coil which has 1000 turns and carries a current of 5A. The length of the magnetic path is 20 metres and flux is 10 milli wb. | CO2 | 5 |
|  | b. | Explain the self inductance with necessary equations. Also derive the relationship between self-induced emf and self inductance. | CO1 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Compare Magnetic and Electric Circuits. | CO2 | 10 |
|  | b. | State the Faraday’s Law of Electromagnetic Induction with necessary equations. | CO2 | 5 |
|  | c. | Find the co-efficient of coupling (K), if the values of Mutual and self inductances are M = L1 = L2 = 2Henry, respectively. | CO2 | 5 |
| 5. | a. | Illustrate the power generating mechanism in a thermal power generating station with neat diagram. | CO4 | 15 |
|  | b. | Compare overhead and underground distribution system. | CO4 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Sketch the block diagram of a Wind Power Generation System. | CO4 | 5 |
|  | b. | State the need for the following components in a nuclear power plant: i.) Moderator ii) Control rod | CO4 | 5 |
|  | c. | Estimate the energy consumed per month by the following electrical appliances. Appliance; Power Rating; No. of item; Hours of Operation / day. LED; 12W; 5; 5. Ceiling Fan; 55W; 2; 6. Mixer; 750W; 1; 0.5. Refrigerator; 500W; 1; 10. | CO4 | 10 |
| 7. | a. | A 6 pole lap wound DC Motor which has 1000 conductors. The flux / pole is 10 milliwebers and the rotating speed is 600rpm. Find the back emf induced in the armature. | CO3 | 5 |
|  | b. | With neat diagrams explain the construction and working principle of DC Generator. | CO3 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Elucidate the working principle of Transformer with neat diagram. | CO3 | 10 |
|  | b. | List out the applications of Three Phase Induction Motor. | CO3 | 5 |
|  | c. | Classify DC Generator based on its field winding. | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the operation of Stair-case wiring with neat diagram. | CO6 | 10 |
|  | b. | Draw the diagram of attraction type MI Instrument. | CO5 | 5 |
|  | c. | Brief out Earthing. | CO6 | 5 |