Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17EE3016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WIND ENERGY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | List the parts of wind turbine and discuss each of the component groups. | CO1 | 10 |
|  | b. | A small wind machine, with a diameter of 6 m, is rated at 4 kW in an 11 m/s windspeed. The installed cost is $10,000. Assuming Weibull parameters at the site to be c = 8m/s and k = 2.2, it is claimed that the capacity factor is 0.38.The interest rate is 11% and the period of the loan is 15 years. Find the cost per unit area, per kW, and the levelized cost per kWh. You do not have to include factors such asinflation, tax credits, or operation and maintenance costs. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A 40 m diameter, three bladed wind turbine produces 700 kW at a wind speed (hubheight) of 14 m/s. The air density is 1.225 kg/m3. Find: - The rotational speed (rpm) of the rotor at a tip-speed ratio of 5.0. - What is the tip-speed (m/s)? - If the generator turns at 1800 rpm, what gear ratio is needed to match the rotor speed to the generator speed? - What is the efficiency of the wind turbine system (including blades, transmission,shafts, and generator) under these conditions? | CO1 | 5 |
|  | b. | Highlight the significance of wind turbine power curve. | CO1 | 7 |
|  | c. | Present a brief overview of some machine elements that are often found in wind turbine applications. | CO1 | 8 |
| 3. | a. | Before wind turbines can be installed, the most appropriate location or locations for them needs to be determined. Outline the wind turbine siting, installation and operational issues. | CO3 | 10 |
|  | b. | Analysis of time series data for a given site has yielded an average velocity of 6 m/s. It is determined that a Rayleigh wind speed distribution gives a good fit to the wind data. - Based on a Rayleigh wind speed distribution, estimate the number of hours that thewind speed will be between 9.5 and 10.5 m/s during the year. - Using a Rayleigh wind speed distribution, estimate the number of hours per year thatthe wind speed is equal to or above 16 m/s. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Two wind turbines emitting 105 dB(A) at the source are located 200 m and 240 maway from a location of interest. Calculate the sound pressure level (in dB(A)) at the point of interest from the combined acoustic effect of the two turbines. Assume a sound absorption coefficient of 0.005 dB(A)/m. | CO3 | 5 |
|  | b. | Review the current technology, support structures, wind resource and the state of offshore wind energy. | CO3 | 10 |
|  | c. | Write a note on wind measurement and instrumentation. | CO3 | 5 |
| 5. | a. | The blades of a wind turbine are ready to be installed on a turbine on top of a ridge.The horizontal blades are supported at each end by saw horses, when a storm front arrives.The turbine crew huddles in their truck as the rain starts and the wind picks up, increasingeventually to 26.82 m/s (60 mph). Realizing that the wind coming up the western slope ofthe ridge roughly follows the 10 degree slope, the field engineer performs a quick calculation and drives his truck upwind of the blades to disrupt the airflow around theblades, preventing them from being lifted by the wind and damaged.The blades are 4.57 m long, 0.61 m wide, and have a mass of 45.36 kg. As the front arrives the temperature drops to 21.2°C. Assume that theblades are approximately symmetric airfoils (the engineer remembered that potential flowtheory predicts that, prior to stall, the lift coefficient of a symmetric airfoil isapproximately: (Cl= 2πsinα). Assume that the center of both the lift and the drag force isconcentrated over the center of mass of the blade and that the leading edge is facing intothe direction of the wind. Assume the air density is 1.20 kg/m3. - Was there a reason to be concerned? At what wind speed will the blades be lifted bythe wind, assuming that there is no drag? - If they are lifted by a 26.82 m/s (60 mph) wind, how fast will they be acceleratedhorizontally, if the blade's lift to drag ratio, Cd/ Cl, is 0.03? | CO4 | 8 |
|  | b. | Determine the power from an ideal turbine rotor using the one dimensional momentum theory and derive the Betz Limit. | CO4 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | Derive the Strip Theory for a generalized rotor, including wake rotation. | CO4 | 10 |
|  | b. | - Wind turbine blades use airfoils to develop mechanical power. Discuss with a neat diagram the airfoil terminology and the forces and moments on an airfoil section. - The operating conditions found at two different points of a blade on a wind turbine are as below: Location (r/R); Wind speed at the blade (m/s); Chord (m); Angle of Attack (degrees). 0.15; 16.14; 1.41; 4.99. 0.95; 75.08; 0.35; 7.63. These conditions were determined at 0°C (32°F), for which the kinematic viscosity is1.33× 10-5 m2/s. What are the Reynolds numbers found at each blade section? Find the solidity of the blade sections if the number of blades in 3 and R=5 m. | CO4 | 6+4 |
| 7. | a. | A 48 V DC wind turbine is hooked up to charge a battery bank consisting of four 12 Vbatteries in series. The wind is not blowing, but the batteries must supply 2 loads. One is alight bulb, rated at 175 W at 48 V; the other is a heater, rated at 1000 W at 48 V. The loads are in parallel. The batteries may be considered to be constant 12 V voltage sources, withan internal series resistance of 0.05 Ohm each. How much power is actually supplied to the loads? How does that power compare to that which is expected? (Ignore the effect oftemperature on resistance of filament and heating elements.) | CO5 | 5 |
|  | b. | Review the operation of different electrical machines used in wind turbines. | CO5 | 10 |
|  | c. | Write a note on transmission and distribution network interfaces. | CO5 | 5 |
| (OR) |  |  |  |  |
| 8. |  | Detail the power quality characteristics of wind turbines and wind farms. Add a note on active and reactive power control. | CO5 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Consider a wind turbine generator connected to a grid system (see Figure) with a line-neutral system voltage, VS. The voltage at the wind turbine, VG, is not necessarily the same as VS. The distribution system resistance is R, and the distribution system reactance is X. The real generated power is P and the reactive power consumed by the wind turbine generator is Q. Suppose R = 8.8 Ω, X = 4.66 Ω, Vs = 11 kV, P = 313 kW, Q = 162 kVAr and the power factor at the generator is pf = 0.89. Determine the voltage difference between the grid and the generator, the voltage drop as a percentage of the grid voltage, the fault level at the generator and the installed generator capacity as a percent of the fault level. | CO6 | 10 |
|  | b. | To successfully generate power using various components, wind turbines need a control system that ties the operation of all the subsystems together. With neat diagrams, discuss in detail the wind turbine control systems. | CO6 | 10 |