Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17EE3017 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PV SYSTEM DESIGN AND INSTALLATION | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the importance of tilt angle for extracting the maximum energy from the solar panel with neat diagrams. | CO2 | 10 |
|  | b. | Design a 24V system with four 12V, 3.5A PV modules and battery capacity of 6V DC, 350Ah. Also find total voltage, total current of solar panel and total Amp-hours of battery. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | With relevant diagrams, elucidate the types of Photovoltaic systems in detail. | CO1 | 15 |
|  | b. | Describe the various photovoltaic system components in detail. | CO1 | 5 |
| 3. | a. | A homeowner in a remote mountain area wants to power the television and refrigerator in his cabin with a photovoltaic system. His refrigerator is a 17 cubic foot model, rated at 500 watts (230V, AC 2.17 Amps). He has timed its operation and says it runs 45 minutes each hour (75 percent of the time). The cabin owner’s small television is rated at 20 Watts (12V DC, 1.67Amps). At first, he admits to watch only one hour of television per day, but upon further questioning, he realizes his first estimate did not account for an additional hour each day of news, weather and sports. Estimate the house owner’s load using load estimation worksheet. | CO3 | 10 |
|  | b. | Define fill factor. Explain the importance of fill factor and efficiency during shading of photovoltaic modules. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | With the help of I-V graph of PV module, explicate the effect of solar radiation and temperature on the Solar Photovoltaic panel. | CO2 | 10 |
|  | b. | List down the factors to be considered when specifying and installing battery storage system for a standalone photovoltaic system. Explain in detail. | CO3 | 10 |
| 5. | a. | The occupants of a remote home are designing a photovoltaic system to meet their 1080 watt hour per day AC electrical load. They have decided on a 12 volt direct current system and feel they need two days autonomy. The maximum depth of discharge they desire over that two day period is 50 percent. The occupants have tentatively selected the Model A battery from ABC Manufacturer, a 6 volt battery rated at 200 amp-hours. Find number of batteries in series and parallel. And also draw the wiring diagram for the above problem. | CO3 | 10 |
|  | b. | Discuss the various types of PV controller used in PV systems in detail. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. |  | Design a standalone PV system for following problem with wiring diagram. A house has the following electrical appliance usage: - Four 18 Watt fluorescent lamp with electronic ballast used 4 hours per day. - Two 60 Watt fan used for 3 hours per day. - One 75 Watt refrigerator that runs 24 hours per day with compressor run 12 hours and off 12 hours. - One 130 Watts TV used for 4 hours per day. - One 1400 Watts Microwave oven used for 5 minutes per day. - One 40 Watt laptop computer used for 8 hours per day. The system will be powered by 12V DC, 85 W PV module with 5.02 peak amps and short circuit amps as 5.34. DC system voltage=48V Battery Depth of discharge=50% Battery rating=12V, 350Ah Days of Autonomy=4 Days (include for all design) Controller voltage is 48V Maximum pass through Amperage=40A Inverter Efficiency=90% Continuous Power output=4000A, 48V nominal Surge Capacity 95 Amps AC For other data, assume as per NEC standard | CO4 | 20 |
| 7. | a. | Explain in brief about the types of inverters used in PV system installation. | CO4 | 10 |
|  | b. | Discuss the importance of equipment grounding and system grounding in PV system Installation. | CO6 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Design a Grid tied PV system with wiring diagram. Size a system for the following home: Yearly Average kWH consumption: 3000 KWh/year Location: Karunya Nagar, TN Average peak sun shine hours per day: 7 Hours System Specifications: Percentage of power to be generated from PV: 60% Record low temperature: 220C Average high Temperature: 380C PV module Choice: Brand X, Model XYZ STC rated watts: 170W Voc: 30.6V Vmax: 24.6V Isc: 7.38A Imp: 6.93A Inverter Choice: Brand X, Model XYZ CEC efficiency: 93.5% Continuous watt rating: 2500W DC input voltage: 150-450 V DC Assume, PV temperature losses=0.88 Derate factor=0.84 | CO3 | 14 |
|  | b. | Explain the different methods of electric power generating systems integrated in buildings. | CO2 | 6 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the final checklist for a newly installed PV system or as a maintenance assessment for an existing PV system. | CO4 | 15 |
|  | b. | List down the over current protection devices used in the PV system and elucidate the same. | CO6 | 5 |