Reg.No. ____________

End Semester Examination – April / May– 2018

| Code : | 17EE3022 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | POWER QUALITY ISSUES AND MITIGATION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the electromagnetic phenomena in Electric Power System. | CO1 | 10 |
|  | b. | A three-phase unbalanced supply system has following phase voltages, Va=0.920° pu, Vb=1.05240° pu, Vc=0.96120° pu. Find the positive, negative and zero sequence components of supply voltages. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Describe in detail-Long-Duration Voltage Variations. | CO1 | 20 |
| 3. | a. | Describe how utilities can deal with problems related to capacitor-switching transients. | CO3 | 10 |
|  | b. | In three-phase ac mains, there is voltage sag at PCC of 15%, 25 % and 20% on three phases for 15, 25 and 30 cycles respectively. Calculate (a) Detroit Edison Sag Score (SS), and (b) voltage sag lost energy index (VSLEI) of this sag event. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Consider single-phase uncontrolled bridge converter with sinusoidal input supply Vs=110V and constant dc load current of 25A. Calculate (a) the mean output voltage, (b) CF, (c) DF,(d) DPF, (e) PF, and (f) %THD. | CO2 | 20 |
| 5. |  | Develop a system model for harmonic simulation studies of the transformer connections. | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A single-phase load (Z=2.5+j1.25 pu) has an input ac voltage of 220 V, 50Hz, AC supply and base impedance of 9.15 ohms per phase. It is to be realized as unity power factor load on ac supply system using PWM based DSTATCOM. Calculate (a) the value of compensator current, (b) its VA rating (c) supply current and (d) equivalent resistance (in ohms) of the compensated load. | CO5 | 20 |
| 7. |  | Illustrate system power quality monitoring concept with monitoring at the substation and selected customer locations. | CO4 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Before embarking on any power quality monitoring effort, one should clearly define the monitoring objectives-Summarize. | CO5 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Represent the control algorithm of the DVR in which a synchronous reference frame theory is used for reference signals estimation. | CO6 | 20 |