Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17EE3053 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BIOMASS ENERGY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Clearly mention the merits and demerits of using biomass for generating energy. | CO3 | 8 |
|  | b. | Elaborate on various biomass resources available. | CO3 | 8 |
|  | c. | List down the factors to be considered in selecting the plant species for energy plantation. | CO1 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Integrate the chemical constituents of biomass fuel based on - As received basis - Air dry basis - Dry basis - Dry and ash free basis | CO1 | 8 |
|  | b. | Discuss on the physiochemical properties of biomass and its measurement techniques. | CO1 | 12 |
| 3. | a. | Classify the different types of Biogas plants. | CO2 | 6 |
|  | b. | Elaborate on the construction and working of Deen Bandhu Biogas plant. Also mention how it is different from Janta Biogas plant. | CO2 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | With relevant diagrams, explain the construction and working of a domestic biogas plant for water hyacinth. | CO2 | 8 |
|  | b. | Mention the problems that are faced in straw fermentation. | CO1 | 4 |
|  | c. | Compare and contrast on the Wet and Dry fermentation. | CO1 | 8 |
| 5. |  | A Farmer has four dairy cows that weigh 1200 kg. The cows are confined so that all the manure can be collected into the digester. The cows would produce about 12.5kg dry solid per day and 10 kg of volatile solids. The volume of the manure and urine would be about 90 litres per day. As 10 litres of water from cleaning are added, the total volume would be 100 litres per day. Assuming methane gas yield as 0.2 per kg, estimate the methane gas production and design digester for the different values of temperature (15, 20, 35⁰C) and different values of retention time (10, 20, 30 days). | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Propose some techniques for maintaining the biogas production and explain them briefly. | CO6 | 6 |
|  | b. | Show how the form of the biogas plant is determined by the digester/gas holder ratio. | CO4 | 6 |
|  | c. | Demonstrate the dependence of shape of the biogas plant on retention time on a floating drum plant and fixed dome plant with relevant diagrams. | CO4 | 8 |
| 7. | a. | Elaborate on the chemistry of the gasification process. | CO4 | 12 |
|  | b. | Point out the technical and commercial barriers in the development of gasifier technology. | CO4 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | With neat sketches, explain the three main designs of fixed bed gasifiers. | CO4 | 12 |
|  | b. | Mention the potential applications of the gasifier based on the total energy requirements for a particular application. | CO4 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | With block diagrams, enlighten on the production of ethanol from wood and sugarcane. | CO5 | 10 |
|  | b. | Discuss on the electricity production from plant wastes. | CO6 | 10 |