Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17EI3028 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # EMBEDDED AUTOMOTIVE SYSTEMS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Identify the major components of automobile Engine. Justify how Electronics play a main role in fuel control, ignition and exhaust system. | CO1 | 10 |
|  | b. | Indicate the desirable characteristics of Exhaust Gas Oxygen Sensor and two types of EGO sensors used for feedback engine control. Explain. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Outline the parameters that affects the engine performance. Discuss the effect of air/fuel ratio on performance of engine. | CO1 | 10 |
|  | b. | Sensors and actuators are the crucial component in automobile control system. Justify how these availability dictates the design of the typical electronic engine control system. | CO2 | 10 |
| 3. | a. | Identify the components of electronic steering module and explain its operations. | CO2 | 10 |
|  | b. | What is Adaptive cruise control? Discuss various information flow signals used in adaptive cruise control system. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Enumerate the components of electronic suspension system and explain its operation. | CO2 | 10 |
|  | b. | What is an actuator? Discuss different types of electrical actuators in automotive applications. | CO2 | 10 |
| 5. | a. | Identify the requirements of ignition system. Explain the different types of ignition system used to improve the engine performance and control. | CO4 | 14 |
|  | b. | Outline the advantages of electronic ignition system with conventional inductive system. | CO4 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Illustrate the working of constant energy ignition system and analyze the input and output speed characteristics. | CO4 | 10 |
|  | b. | What is ideal ignition timing? List the Factors that affect the maximum spark advance before the onset of knock. | CO3 | 10 |
| 7. | a. | Describe the principles, operation, and construction of solid-state ignition systems. | CO5 | 10 |
|  | b. | # Illustrate various Components of Electronic Ignition System and its working. | CO5 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | With schematic explain the working of distributor less ignition system. | CO5 | 14 |
|  | b. | Summarize the importance of engine management system. | CO5 | 6 |
|  |  | Compulsory: |  |  |
| 9. | a. | How vehicle is being tracked using Global Positioning System and explain the major constituents of the GPS-based tracking. | CO6 | 10 |
|  | b. | With block diagram, explain anti-theft system for bike using GSM. | CO6 | 10 |