Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : 17ME3001 |  | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : FINITE ELEMENT METHODS IN ENGINEERING |  | Max. marks : | 100 |

Answer ALL questions (5 x 20 = 100 MARKS)

| Q. No | Sub. Div | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1 | a | Solve the following system of equation using Choleski decomposition method: x +3y +8z =4 x + 4y +3z = -2 x + 3y + 4z = 1 | CO1 | 10 |
|  | b | Using Power method, find the dominant Eigen value and Eigen vector of given matrix A = 2−10−12−10−12 | CO1 | 10 |
|  |  | (OR) |  |  |
| 2 | a | Elucidate the simultaneous linear algebraic equations using Gaussian Elimination method. X1 – X2 + 3X3 = 10 2X1 + 3X2 + X3 = 15 4X1 + 2X2 – X3 = 6. | CO1 | 10 |
|  | b | Discuss on historical background of finite element method. | CO1 | 5 |
|  | c | Differentiate between FDM, FEM and FVM method. | CO1 | 5 |
| 3 | a | Develop element stiffness matrix for a one dimensional truss element using direct stiffness method. | CO2 | 10 |
|  | b | Consider the bar shown in figure with axial force P = 30KN is applied as shown. Determine the nodal displacement, stresses in each element and reaction forces. | CO2 | 10 |
|  |  | (OR) |  |  |
| 4 | a | Briefly explain the step by step procedure for finite element analysis with a neat flowchart. | CO2 | 10 |
|  | b | The differential equation of a physical phenomenon is given by d2ydx2+500x2=0 0≤x≤1, Trial function, y=a1(x−x4) Boundary conditions are, y (0) = 0; y (1) = 0 Calculate the value of parameter a1 by the following methods: - Point collocation method. - Least square method. - Sub domain method. - Galerkin’s method. | CO2 | 10 |
| 5 |  | Compute the element matrices and vectors for the element shown in figure. When the edges jk and ki experience convection heat loss. | CO4 | 20 |
|  |  | (OR) |  |  |
| 6 |  | Decide the temperature distribution in a one dimensional circular cross section as shown in figure. The fin is circular in shape 8cm long and 1cm diameter. Assume that convection heat loss occurs from the right end of the fin. (use 2 element idealization) L=8 cm 100C 1 cm diameter φf=20∘C, h=0.2Wattscm2 ∘C k=2Wattscm ∘C | CO3 | 20 |
| 7 | a | Define the temperature (φ) at the point (P) located at (4,7) where (φi=400C), (φj=350C), and (φk=450C), as shown in the figure below. | CO5 | 6 |
|  | b | Derive a shape function for 3 node triangular (CST) element. | CO5 | 14 |
|  |  | (OR) |  |  |
| 8 | a | What is meant by serendipity elements? Explain. | CO5 | 6 |
|  | b | Develop the shape function for eight noded rectangular element in (ξ,η) coordinate system. | CO5 | 14 |
| 9 |  | Compulsory: Determine the natural frequencies and mode shapes of transverse vibration for a beam fixed at both sides. The beam may be modelled by two elements, each of length L and cross-sectional area A. consider lumped mass matrix approach. | CO6 | 20 |