Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 17ME3003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ADVANCED MECHANICAL VIBRATIONS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Obtain the steady state response of a single degree of freedom system shown in Figure 1 , under free vibration subjected to viscous damping. Discuss all the three cases of damping. k m c Figure1 | CO3 | 20 |
| (OR) |  |  |  |  |
| 2. |  | A mass of 10 kg is suspended from one end of a helical spring, the other end being fixed. The stiffness of the spring is 10N/mm. The viscous damping causes the amplitude to decrease to one-tenth of the initial value in four complete oscillations. If a periodic force of 150cos50t N is applied at the mass in the vertical direction, find the amplitude of forced vibrations? What is the value of resonance? | CO6 | 20 |
| 3. |  | Explain in detail the real behaviour of a vibration absorber. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Derive suitable expression for transverse vibrations for a simply supported beam of length l and uniform cross section. | CO1 | 20 |
| 5. |  | Use matrix iteration method to determine the natural frequencies of the system shown in Figure 2. Figure 2. | CO2 | 20 |

| (OR) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 6. |  | Using Holzer’s method, find the natural frequency of the system shown in Figure3. Assume m1=m2=m3=1Kg & K1=K2=K3=1N/m. m1 m2 m x1(t) x2(t) m3 m x3(t) k1 k2 k3 K4 Figure3 | CO2 | 20 |
| 7. |  | With a neat sketch, explain displacement and acceleration measuring instruments. | CO4 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Discuss the International standards for noise measurement and noise reduction at machines. | CO6 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | With example, explain the mechanism of noise generation for any two most commonly used machines in industrial installations. | CO5 | 20 |