Reg.No. ____________

End Semester Examination – April/May – 2018

| # Code : | # 17ME3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED MECHANISM DESIGN | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain Kutzbach criterion for the mobility of planar mechanisms and spatial mechanisms. | CO1 | 4 |
|  | b. | Give an example where Kutzbach criterion fails. Explain why? | CO1 | 2 |
|  | c. | # Determine the mobility of the following mechanisms. # (i) # (ii) | CO1 CO1 | 7 7 |
| (OR) |  |  |  |  |
| 2. | a. | What is Grashof’s law? Explain Grashof’s kinematic chain I and II. | CO1 | 10 |
|  | b. | Explain the four inversions of slider-crank mechanism with examples. | CO1 | 10 |
| 3 | a | # Derive an analytical solution of output angle θ4 of a four bar crank rocker linkage in terms input crank angle θ2 and link lengths. | CO2 | 10 |
|  | b | Derive an analytical solution of velocity of an offset slider-crank mechanism in terms of input crank angle θ2 , coupler angle θ3 and link lengths. Crank rotates with uniform angular velocity ω2 . | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | # Apply Kennedy’s rule to find the instantaneous centres of the crank-slider mechanism shown in part b. | CO3 | 6 |
|  | b. | # Angular velocity of crank AB is 600 r.p.m. Dimensions are given in mm. Find the velocity of point B, C and mid-point of links AB, BC and CD using instantaneous centre method. AD is ground link. | CO3 | 14 |
| 5. | a. | Sketch the locus of a moving and fixed centrode of a four-bar crank- rocker mechanism | CO3 | 8 |
|  | b. | # Derive Euler Savary equation. | CO3 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | # What is structural error? What is mechanical error? What is branch defect? What is order defect? | CO4 | 8 |
|  | b. | # Explain Chebychev spacing to generate the function y =x0.9 over the range 1≤ x ≤4 using three precision positions and graphically verify the results. | CO4 | 12 |
| 7. | a. | Define path generation, function generation and motion generation | CO4 | 4 |
|  | b. | # Synthesize a function generator to solve the equation y =1/x over the range 1≤ x ≤2 using three precision positions. Choose starting angle for input link as 30° and total swing angle as 90°. For output link choose starting angle as 240° and total swing angle as 90°. | CO4 | 16 |
| (OR) |  |  |  |  |
| 8 |  | A four-bar mechanism is subjected to external force and torque as shown below. Draw free body diagram of each link from the given four bar linkage. Determine the torque T on the input link O2A for static equilibrium. Also find the forces on the bearings O2, A, B and O4. | CO5 | 18 |
|  | b. | State and explain D’Alembert’s principle of dynamic equilibrium. | CO5 | 2 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain Denavit – Hartenberg parameters. | CO6 | 10 |
|  | b. | Explain transformation-matrix of position analysis and its transformation form of the loop-closure equation. | CO6 | 10 |