Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17ME3010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF MECHANICAL SYSTEM ELEMENTS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Enumerate the methods of determining stresses and its working principles. | CO1 | 15 |
|  | b. | Show a relationship between berry and huggenberger type strain gauge in terms of accuracy and applications. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. |  | Illuminate the stresses in various components of circular ring, cylinder and sphere. | CO5 | 20 |
| 3. |  | Itemize the stress concentration at a variable thickness transition section in a cylindrical vessel. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Classify stress concentration factor with respect to superposition, dynamic and thermal transient conditions. | CO1 | 12 |
|  | b. | Describe localized stresses and their significance. | CO1 | 8 |
| 5. | a. | Correlate fatigue and stress concentration factor. | CO1 | 12 |
|  | b. | Write short notes on the properties of brake lining materials. | CO5 | 8 |
| (OR) |  |  |  |  |
| 6. |  | Design a pressure vessel in weldment perspective. | CO5 | 20 |
| 7. | a. | Classify and explain the major transport category and their applications. | CO2 | 8 |
|  | b. | It is required to determine the size of the rope for infinite life. The weight of the hoist and the material to be lifted is 6 kN. The load has to be raised from a depth of 100 m. The maximum speed of the rope is 6 m/s and it is attained in 6 sec. Use a 6 x 19 rope with σ u = 1600 N/mm 2 . Also determine the working factor of safety. Take E T = 0.8 x 10 5 N/mm 2 . | CO4 | 12 |
| (OR) |  |  |  |  |
| 8. |  | Briefly explain the design principles of Pneumatic and vibratory conveyor. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | The hydraulic cylinder 400mm bore operates at a maximum pressure of 5 N/mm2. The piston rod is connected to the load and the cylinder to the frame through hinged joints. Design 1. Cylinder 2. Piston rod 3. Hinge pin and 4. Flat end cover. The allowable tensile stress for cast steel cylinder and end cover is 80 MPa and for piston rod is 60MPa. Draw the hydraulic cylinder with piston, piston rod, end cover and O ring. | CO4 | 20 |