Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17ME3018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THEORY OF METAL CUTTING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | What is chip thickness ratio? | CO1 | 5 |
|  | b. | The orthogonal cutting of steel is done with 100 rake tool with a depth of cut of 2 mm and feed rate of .20mm/rev.The cutting speed is 200m/min.The chip thickness ratio is .31.The vertical cutting force is 1200N and the horizontal cutting force is 650N.Calculate i)the friction angle.ii)friction force along rake face iii)normal force on the shear plane iv)shear force and validate the shear angle using Merchants theory. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | # Compare and evaluate orthogonal and oblique cutting. | CO1 | 10 |
|  | b | # With neat sketches explain BUE and continous chips. | CO1 | 10 |
| 3. | a. | Explain the tool nomenclature as used in ASA systems with neat sketch. | CO2 | 14 |
|  | b. | List the nomenclature of a drilling tool with a neat sketch. | CO2 | 6 |
| (OR) |  |  |  |  |
| 4. |  | Derive the force analysis during oblique cutting. | CO1 | 20 |
| 5. | a. | Enumerate a few types of cutting fluid . | CO4 | 5 |
|  | b. | Elaborate on the applications of cutting fluids.What are the points from which cutting fluid maybe applied in machining operations? | CO4 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | Explain any tool work thermocouple method of measuring temperatures in metal cutting. | CO4 | 15 |
|  | b | List the zones in metal cutting where the heat is generated. | CO4 | 5 |
| 7. | a. | Explain about the existing tool materials. | CO5 | 15 |
|  | b. | Define Taylor’s tool life equation. | CO5 | 5 |
| (OR) |  |  |  |  |
| 8. |  | Discuss in detail the different types of wear that take place in cutting tool. Elaborate on any three of them along with the mechanism involved. | CO5 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | List the components of cost of machining a given workpiece. | CO3 | 5 |
|  | b. | Evaluate the various restrictions in optimum cutting conditions. | CO3 | 15 |