Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17ME3026 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED HEAT TRANSFER | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Consider a 0.8 m high and 1.5 m wide glass window with a thickness of 8 mm and a thermal conductivity of k = 0.78 W/m-K. Determine the steady rate of heat transfer through this glass window and the temperature of its inner surface for a day during which the room is maintained at 200C where the temperature of the outdoors is -100C. Take the heat transfer coefficients on the inner and outer surfaces of the window to be h1 = 10 W/m2-K and h2 = 40 W/m2-K, which includes the effect of radiation. | CO1 | 15 |
|  | b. | Explain the concept of critical radius of insulation on a small diameter wire. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Derive the general heat conduction equation in rectangular coordinates. | CO1 | 10 |
|  | b. | Derive an expression for temperature distribution of an infinitely long pin. | CO1 | 10 |
| 3. |  | # A system for heating water from an inlet temperature of 200C to an outlet temperature of 600C involves passing the water through a thick walled tube having inner and outer diameters of 20 and 40 mm. The outer surface of the tube is well insulated, and electrical heating within the wall provides for a uniform heat generation rate of 106 W/m3. # For a water mass flow rate of 0.1 kg/s, how long must the tube be to achieve the desired outlet temperature? - If the inner surface temperature of the tube is 700C, what is the local convection heat transfer coefficient at the outlet? | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Air at 300C is flowing across a tube with a velocity of 25 m/s. The tube could be either a square with a side of 5 cm or a circular cylinder of diameter 5 cm. Compare the rate of heat transfer in each case if the tube surface temperature is 1240C. | CO2 | 15 |
|  | b. | Discuss briefly the development of velocity boundary layer for flow through a vertical plate. | CO2 | 5 |
| 5. |  | A cylindrical furnace has outer radius 1m and height 1m. The top and bottom surface of the furnace have emissivities ε1 = 0.8 and ε2 = 0.4 respectively, and are maintained at temperatures T1= 700 K and T2 = 500 K. The side body closely approximates a black body and is maintained at a temperature of 400 K. Determine the net rate of radiation heat transfer at each surface during steady operation and explain how these surfaces can be maintained at specified temperatures. | CO6 | 20 |
| (OR) |  |  |  |  |
| 6. |  | # Consider the 5m X 5m X 5m cubical surface, whose surfaces closely approximate black surfaces. The base, top and side surfaces of the furnace are maintained at uniform temperatures of 800 K, 1500 K, 500 K respectively. Determine # The net rate of radiation heat transfer between the base and the side surfaces, # The net rate of radiation heat transfer between the base and the top surface, and - The net rate of radiation heat transfer from the base surface. | CO6 | 20 |
| 7. | a. | Water is boiled at atmospheric pressure by a horizontal polished copper heating element of diameter 5mm and emissivity 0.05, immersed in water. If the surface temperature of the heating wire is 3500C, determine the rate of heat transfer from the wire to the water per unit length of the wire. | CO4 | 15 |
|  | b. | Write short notes on drop wise condensation. | CO4 | 5 |
| (OR) |  |  |  |  |
| 8. |  | A refrigerator is designed to cool 250 kg/h of hot liquid of specific heat 3350 J/kgK at 1200C using a parallel flow arrangement. 1000 kg/h of cooling water is available for cooling purposes at a temperature of 100C. If the overall heat transfer coefficient is 1160 W/m2 K and the surface area of the heat exchanger is 0.25 m2, calculate the outlet temperatures of the cooled liquid and water and also the effectiveness of the heat exchanger. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | The water in a 5 m X 15 m outdoor swimming pool is maintained at a temperature of 270C.The average ambient temperature and relative humidity are 270C and 40 % respectively. Assuming a wind speed of wind speed of 2 m/s in the direction of the long side of the pool estimate the mass estimate the mass transfer coefficient for the evaporation of water from the hot surface. | CO5 | 15 |
|  | b. | Explain briefly the various modes of mass transfer. | CO5 | 5 |