Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 17ME3031 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED INSTRUMENTATION IN THERMAL ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A resistor has a nominal stated value of 10 Ω±1 percent. A voltage is impressed on the resistor, and the power dissipation is to be calculated in two different ways: (1) from P = E2/R and (2) from P = EI. In (1) only aVoltage measurement will be made, while both current and voltage will be measured in (2).Calculate the uncertainty in the power determination in each case when the measured values of E and I areE = 100 V ± 1% (for both cases) I = 10 A ± 1% | CO1 | 10 |
|  | b. | Describe the role of uncertainty analysis in experiment planning. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | The resistance of a certain size of copper wire is given as R = R0[1 + α(T − 20)] where R0 = 6 Ω± 0.3 percent is the resistance at 20◦C, α = 0.004◦C−1 ± 1 percent is the temperature coefficient of resistance, and the temperature of the wire is T = 30±1◦C. Calculate the resistance of the wire and its uncertainty. | CO1 | 15 |
|  | b. | Define Errors. Describe the various types of errors and its classifications. | CO1 | 5 |
| 3. |  | Briefly discus about the importance of Transducers and types. And explain the working principle LVDT transducer and Digital displacement transducer. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Explain the working principle of Dead Weight Tester and the Diaphragm and Bellow gates with a neat sketch. | CO2 | 20 |
| 5. |  | Briefly discuss about the Temperature measurement by Radiation and explain the working principle of optical pyrometer with a neat sketch. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the measurement of surface tension and explain its type with a suitable diagram. | CO4 | 10 |
|  | b. | Discuss in detail about Radiation properties of surfaces. | CO4 | 10 |
| 7. |  | Explain in detail of gas chromatograph and smoke meters with a neat sketch. | CO5 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Describe the process and the working principle of non-dispersive infrared analyser with a neat sketch. And explain the working principle of Flame ionization detector with suitable diagram. | CO5 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Briefly discuss about the data transmission and explain the working principle of Digital-to-Analog (D/A) and Analog-to-Digital (A/D) converter. | CO6 | 20 |