Reg.No. _____________

End Semester Examination – Nov/Dec – 2018

| Code : | 18AT2034 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | FARM MACHINERY AND POWER | Max. marks : | 100 |

| Q. No. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- |
|  | PART-A(20X1=20 MARKS) |  |  |
| 1. | Define farm power. | CO1 | 1 |
| 2. | An average man can develop maximum of about ____________. | CO1 | 1 |
| 3. | Give exmples for mobile farm power. | CO1 | 1 |
| 4. | Define –Scavenging. | CO2 | 1 |
| 5. | Compression ratio of diesel engine is __________. | CO1 | 1 |
| 6. | Define Piston. | CO1 | 1 |
| 7. | Brief about the uses of Piston ring. | CO2 | 1 |
| 8. | Define stroke. | CO1 | 1 |
| 9. | Mention the components of lubrication system. | CO1 | 1 |
| 10. | Mention the factors affecting rate of cooling. | CO3 | 1 |
| 11. | Mention the function of clutch. | CO3 | 1 |
| 12. | Mention the use of differential lock. | CO3 | 1 |
| 13. | Mention the cost involved in the fixed cost. | CO1 | 1 |
| 14. | Mention objectives of the tillage. | CO1 | 1 |
| 15. | Define puddling. | CO3 | 1 |
| 16. | Define broadcasting. | CO3 | 1 |
| 17. | Define ULV. | CO2 | 1 |
| 18. | Define registration. | CO1 | 1 |
| 19. | Define Knife section and knife head. | CO1 | 1 |
| 20. | Define concave clearance. | CO1 | 1 |

|  | PART B(10 X 5= 50 MARKS) (Answer any 10 from the following) |  |  |
| --- | --- | --- | --- |
| 21. | Explain merits and demerits of farm power sources. | CO1 | 5 |
| 22. | Explain the comparison between two stroke and four stroke engine. | CO3 | 5 |
| 23. | Explain about Radiator and its working principle with neat sketch. | CO3 | 5 |
| 24. | Explain about Continous ploughing method and round ploughing method. | CO1 | 5 |
| 25. | Explain the working principle of differential with neat sketch. | CO2 | 5 |
| 26. | Explain about mould board types with neat sketch. | CO1 | 5 |
| 27. | Explain the working principle of Motorized knapsack sprayer. | CO3 | 5 |
| 28. | a. Cotton seeds are to sown in 75 cm row spacing by a 4-row planter having 60 cm effective drive wheel. A 12-teeth sprocket on planter shaft is driven by 10-teeth sprocket on the axle. The seed plate has 12 cells and each cell meters 2 seeds. Seed plate is operated by a set of 25 teeth ring gear and 15 teeth pinion gear. Determine the seed spacing and seed population per hectare. b. A sunflower planter is required to have a plant population of 55,000 plants/ha at a row to row spacing of 60 cm and plant to plant spacing of 30 cm. The diameter of ground wheel is 40 cm with overall transmission ratio of 0.5. Determine the number of cells on the periphery of plate. | CO2 | 5 |
| 29. | Explain about components of seed drill. | CO3 | 5 |
| 30. | Explain about i) Tandem and ii) Off-set disc harrow. | CO1 | 5 |
| 31. | Compare Disc Plough and MB-Plough. | CO3 | 5 |
| 32. | a. A 5 x 20 cm double action disc harrow is operated by a tractor having a speed of 5 km/h. Calculate the actual field capacity , assuming the field efficiency of 80 percent. b. An indigenous plough has a 20 cm wide furrow at the top and 10 cm depth. Calculate the volume of soil handled per day 8 hours if the speed of working is 2.5 km/h. | CO2 | 5 |

|  | PART C(2 X 15= 30 MARKS) (Answer any 2 from the following) |  |  |  |
| --- | --- | --- | --- | --- |
| 33. | a. | Explain Four stroke engine working principle with neat sktch. | CO3 | 7 |
|  | b. | Explain the working principle of governor. | CO3 | 8 |
| 34. | a. | Explain the seed drill calibration procedure. | CO1 | 7 |
|  | b. | Explain the adjustment in tractor-drawn mould board for efficient operation. | CO2 | 8 |
| 35. | a. | Explain the components of combine harvester with neat sketch. | CO2 | 7 |
|  | b. | The initial cost of 35 hp Massy Ferguson Tractor owned by a farmer is Rs. 3,00,000/-. The tractor is expected to work for 10 years. In a year the farmer uses the tractor for 1000 hours. The farmer also owns a 11 tyned cultivator. The tynes are spaced at 20 cm apart. The cost of the cultivator is Rs.12,000/-The tractor consumes 3 litres of diesel while ploughing with the cultivator. The life of the cultivator is 10 years. The farmer uses the cultivator for 400 hours in a year. The cultivator is operated at a speed of 4 km/h. Calculate the cost of ploughing 2 ha of land with the cultivator. Assume all other necessary data. | CO3 | 8 |

Key

- Farm Power is the basic energy requirement for agricultural operations.

- 0.1 hp

- Human (men, women, children) , Draught animals (bullocks, buffaloes, camels, horses and ponies, mules and donkeys), Tractors,Power tillers and Self propelled machines

- The process of removal of burnt or exhaust gases from the engine cylinder is known as scavenging.

- 14:1 to 22:1

- It is a cylindrical part closed at one end which maintain a close sliding fit in the engine cylinder.

- It forms a gas tight combustion chamber for all positions of piston.

It reduces contact area between cylinder wall and piston wall preventing friction losses and excessive wear.

It controls the cylinder lubrication.

It transmits the heat away from the piston to the cylinder walls.

- It is the linear distance traveled by the piston from Top dead centre (TDC) to Bottom dead centre (BDC).

- oil pump,  oil filter,  Pressure regulating valve, By pass valves, Breather system and  oil coolers.

- Thermal conductivity,Surface area,Viscosity, Temperature differences between atmosphere and heated surface.

- It connects or disconnect the engine from transmission system

- To increase or decrease the speed of the rear wheels invidually

- depreciation, interest on investment, taxes, insurance and housing

- to obtain deep seed bed, suitable for different type of crops.,to add more humus and fertility to soil by covering the vegetation.,to destroy and prevent weeds, to aerate the soil for proper growth of crops., to increase water absorbing capacity of the soil, to destroy the insects, pests and their breeding places and  to reduce the soil erosion.

- Puddling is the mechanical manipulation of soil in presence of standing water in the field to create a least pervious layer that prevents the loss of water through percolation

- Broadcasting is the process of random scattering of seed on the surface of seedbeds. It can be done manually or mechanically both.

- Ultra low volume (ULV) spray (less than 5 litres spray/ha).

- A mower knife is said to be in proper registration when the knife section stops in the centre of its guard on every stroke i.e. the centre of the knife section is at the centre of the guard, when it is in operating condition

- Knife section - It is a flat steel plate (triangular shape) with two cutting edges.

Knife head -	It is the portion of the knife which is connected to the pitman.

- It is the clearance between threshing drum and concav